San Francisco Bay Area Data For 
The Analysis of Non-Motorized Travel

March, 2005

USER’S GUIDE

# TABLE OF CONTENTS

1.	INTRODUCTION	1

2.	DESCRIPTION of DATASETS	2

2.1.	Survey Data	3

2.2.	Transportation Network Data	4

2.3.	Polygon Layers	4

2.4.	Urban Form Measures	5

3.	MAPPING ANALYSIS	7

3.1.	Opening the County Map	7

3.2.	Adding the Household Layer	8

3.3.	Locating the Households tha Use Only Non-Motorized Modes	9

3.4.	Thematic Mapping of Walk Trips	10

4.	STATISTICAL ANALYSIS	13

4.1.	Basic Descriptive Statistics	13

4.2.	Cross-Tabulation Analysis	15

4.3.	Joint-Table Analysis	17

Appendix I. Data Dictionary for the Households Dataset	18

Appendix II. Data Dictionary for the Persons Dataset	23

Appendix III. Data Dictionary for the Trips Dataset	27

# INTRODUCTION

This user’s guide will illustrate how to access the GIS-based data compiled for the San Francisco Bay area and will provide a basis for conducting an analysis of non-motorized travel behavior use the data set.

It is assumed that you have TransCAD installed before you proceed with carrying out the exercises in this document.  You also need to have the entire /Data folder copied from the accompanying CD onto your hard drive.

The remainder of the user’s guide is organized as follows.  Section 2 describes the source and the content of each of the datasets provided with this tutorial.  Section 3 presents a set of exercises designed to familiarize you with the different geographical layers that are available and to expose you to some examples of mapping analysis.  Section 4 focuses on the attribute tables provided and describes a set of statistical analysis exercises which you can perform with the data.

# DESCRIPTION of DATASETS

This spatially referenced data base provides TransCAD files describing the zonal configuration, zonal land-use and demographics, urban form, and the network for the San Francisco Bay area.  It also includes the travel and demographic information collected from the 2000 Bay Area Travel Survey.

The datasets are:

| Dataset | Description |
| --- | --- |
| /Survey/Households | Feature dataset displaying point locations of surveyed households |
| /Survey/Persons | Feature dataset displaying point residential locations of surveyed individuals |
| /Survey/Trips | Feature dataset displaying point locations of surveyed trip destinations |
| /Network/Bikeways | Feature dataset displaying linear bicycle network |
| / Network /Highways | Feature dataset displaying linear highway network |
| / Network /LocalRoads | Feature dataset displaying linear local road network |
| /Polygons/County | Feature dataset displaying polygon definitions of counties |
| /Polygons/TAZ | Feature dataset displaying polygon definitions of TAZs |
| /UrbanForm/QuarterMiBands | Feature dataset displaying ¼ mile bands constructed around point locations of surveyed households |
| /UrbanForm/OneMiBands | Feature dataset displaying 1 mile bands constructed around point locations of surveyed households |
| /UrbanForm/FiveMiBands | Feature dataset displaying 5 mile bands constructed around point locations of surveyed households |

## Survey Data

The three datasets under the /Survey directory have been compiled from the 2000 Bay Area Transportation Survey (BATS), originally conducted by MORPACE International Inc. for the Metropolitan Transportation Commission (MTC), California.  The survey collected information on all activity and travel episodes undertaken by individuals over a two-day period from over 15,000 households in the nine counties in Bay Area.  The information collected on activity episodes included the type of activity, start and end times of activity participation, and the geographic location of activity participation.  Travel episodes were characterized by the mode used, and the start and end times of travel.  Furthermore, data on individual and household socio-demographics, individual employment-related characteristics, household auto ownership, household location, and internet access and usage were also obtained.

The Trips table contains, in each record, the attributes describing each travel episode, including purpose, mode, time of day, day of week, number of stops made, and longitude and latitude of the destination location.  The Persons table contains, in each record, each trip makers’ attributes, such as socio-demographics, employment-related characteristics, study-related characteristics, internet access and usage, and geographic coordinates of the home location.  It includes also the aggregate trip counts by mode and by trip purpose for each surveyed individual.  The Households table includes, for each surveyed household, attributes such as household structure, auto ownership level, bicycle ownership level, and geographic coordinates.  It also includes aggregate descriptors about the individuals in each household (such as household structure and age composition) and their trip counts.

For details about the Households, Persons, and Trips tables, please refer to Appendix I, II, and III, respectively.

## Transportation Network Data

Two of the three datasets under the /Network directory, Highways and LocalRoads, were extracted from the Census 2000 TIGER files.  The Highways network file contains all interstate, toll, national, state and county highways in the Bay area.  The LocalRoads network file contains the local, neighborhood, and rural roads in the region.

The Bikeways network was provided by MTC.  It describes all existing bicycle facilities in the Bay Area region, including class 1 (separate paths for cyclists and pedestrians), class 2 (painted lanes solely for cyclists), and class 3 (signed routes on shared roads) facilities.

## Polygon Layers

Two additional geographic layers were provided by the MTC.  The County dataset defines the nine counties in the region.  The TAZ dataset provides, for each of the 1099 internal and external Traffic Analysis Zones, the following information: (1) area coverage by land-use purpose, (2) number of housing units, (3) employment levels by sector, (4) zonal population, income and age distribution of the population, and (5) area type of the zone (central business district, urban, suburban, or rural).

## Urban Form Measures

Bands of 1/4, 1, and 5 mile width have been constructed around the point features in /Survey/Households and stored in /UrbanForm/QuarterMiBands, /UrbanForm/OneMiBands, /UrbanForm/FiveMiBands, respectively.  Data from /Polygons/TAZ, /Network/Bikeways, /Network/Highways, and /Network/LocalRoads were overlaid onto these bands by:

- Assuming that the TAZ attributes follow a uniform distribution within each zone, so that data for a given zone can be disaggregated uniformly over the zone.  For instance, if the number of service employment centers a 10 square-mile zone is 100, then every square-mile area in the zone is assumed to have 10 service employment centers.  The disaggregated data are then projected onto, and re-aggregated over, the circular buffers created around the geo-coded location of each household to produce the corresponding measures for the circular units.

- Projecting each of the network layers onto the circular units and summing up the lengths of the line segments falling within each circular unit.

A number of additional variables were also computed for each band layer.  These included land-use composition measures (percentage of coverage by land-use type) and a more complex measure of land-use diversity defined by:

LUMIXi=1−RiTi−13+CIiTi−13+OiTi−1343

where Ti is the total area of the circular unit i; and Ri, CIi, and Oi are the acreages of residential, commercial and industrial, and other land use types, respectively.  This land-use mix index takes a value between 0 and 1, where 1 indicates perfect mixing of land uses and 0 indicates that the land in a particular area is completely dedicated to a single land use.

The outcome of the above-described process is three sets of urban form measures computed using three band widths around each residence.  These band-based urban form measures are better than the TAZ-based measures for the purpose of analyzing the influence of environmental factors on non-motorized travel behavior.  This is because the band-based measures represent the quality of the environment within explicitly defined walking or cycling distances from each residence, while the spatial extent of the TAZ-based measures would vary from zone to zone due to the irregularity of the zones.

# MAPPING ANALYSIS

## Opening the County Map

- In the TransCAD main menu, choose File-Open or click  on the toolbar, to access the File Open dialog box.

- Navigate to the /Polygons/County directory.

- Choose Geographic File from the Files of Type drop-down list.

- Choose County.

- Click Open to open the file and display the map.  TransCAD displays a map of the nine counties in the Bay area

2

3

4

5

## Adding the Household Layer

- Choose Map-Layers or click  on the toolbar. TransCAD displays the Layers dialog box.

- Click Add Layer to display the File Open dialog box.

- Navigate to the /Survey directory.

- Choose Geographic File from the Files of Type drop-down list.

- Choose Households from the list and click Open.  TransCAD adds the Households Layer in the Layer dialog box.

- Click Close to exit the Layers dialog box.  TransCAD displays the household point locations in the county map.

2

5

## Locating the Households tha Use Only Non-Motorized Modes

2

Choose Selection-Select by Condition from the main menu.  TransCAD displays the Select by Condition dialog box.

- Type TYPE=1 in the Condition box.

- Type Non-motorized HH in the Set Name box.

- Click OK.  The households using only non-motorized modes are highlighted in the map.

- To show only the Non-motorized HH set of the Households layer, choose Selection-Settings from the main menu.

- Make sure that the Households layer is highlighted.

- Click Status so that the Households layer is set to invisible.  Then click Close.

- TransCAD displays a map as the one shown on the right.

- You can examine the characteristics of these households by clicking  on the toolbar.  TransCAD displays the Dataview for the household layer.

- From the toolbar, set the Dataview to the selected households only.

3

6

7

10

## Thematic Mapping of Walk Trips

Thematic maps are maps constructed to emphasize the spatial pattern of one or more spatial variables.  You choose the theme types based on the number of variables you want to show and the types of  variables you wish to present.  Please consult the TransCAD Help file for the different themes available and how these themes from a dataview.  Below, we describe the construction of a color theme to show the spatial distribution of walk-trip ends by trip purpose.

- Open another county map by repeating Exercise 3.1.

- Follow the steps in Exercise 3.2 to add the Trips layer onto  the map.

- Open the Dataview corresponding to the Trips layer by clicking  on the toolbar.

- From the main menu, choose Dataview-Formula Fields.  TransCAD displays the Formula dialog box.

- Enter WALK*TRIP in the Formula box and Walk Trip Purpose in the Formula Fields.  And click OK.

- Choose Map-Color Theme from the main menu.

2

4

7

8

9

In the Color Theme dialog box, select Walk Trip Purpose from the Field list.

- Select List of Values from the Method list.

- Click the Styles tab.  TransCAD will take a second to perform some computation in the background.

- To change the display of any class, simply choose it from the Class list and edit the corresponding style by clicking the Style button.  For example, in order to hide the non-walk trips, select the second item  from the Class list and click Style.

- In the Style dialog box, select the empty icon and click OK.

- With the second class selected, remove 0.0000 from the Legend Text field.

- Go through the remaining items in the Class list and change their corresponding style and legend text as shown in the diagram to the right.

11

13

- Click OK.  TransCAD will display the thematic map of the walk trips, with the four trip purposes (primary job, maintenance shopping, pure recreation, and other) color coded (as shown on the right).  This allows you to examine the spatial distribution of different types of walk trips in the study region.

14

# STATISTICAL ANALYSIS

In TransCAD, the data tables associated with the geographical layers can be used for statistical analysis without involving their geographic definitions.  In this section, we provide a few examples of how to conduct such analysis.

## Basic Descriptive Statistics

The first example involves examining the attributes in the Persons table.

2

Choose File-Open from the main menu to access the File Open dialog box.

- Navigate to the /Survey directory.

- Choose dBase File from the Files of Type drop-down list.

- Choose Persons.dbf and click Open .

- Click No if you see a confirmation box asking whether you want to creat a geographic file.

- TransCAD displays the Persons table in a dataview window.

3

4

- Choose Dataview-Statistics or click  on the toolbar. TransCAD displays the Save Statistics Table As dialog box.

- Type a file name for the statistics table and click Save.  TransCAD computes the statistics and displays the results in a new dataview as shown on the right.

- The statistics table shows the range of values corresponding to each variable in the Persons table.  For example, the number of bicycle trips per person (BIKEPER) ranges from 0 to 13.

## Cross-Tabulation Analysis

Below, we show an example of using the Persons table to examine the relationship between gender and the use of the bicycle mode.

- Open the Persons dataview following the steps 1 to 5 in Exercise 4.1.

- Choose Statistics-Tabulations from the main menu.

- In the Tabulations dialog box, set Field 1 to FEMALE and Method to List of Values.  Also, set #Classes to 2 (this is because FEMALE is a dummy variable which takes the value of either 0 or 1).

- Click the Second Field tab.

- Set Field 2 to BIKEPER and Method to List of Values.  Also, set #Classes to 5.  (As we find from Exercise 4.1, the variable BIKEPER takes a value from 0 to 13.  By setting the number of classes to 5 and the aggregation method to List of Values, you aggregate the values into five classes: 0, 1, 2, 3, 4, and >4.)

38

4

5

- Click OK.  TransCAD will bring up a Save As dialog box, in which you should specify the path where you would like the tabulation result be stored.

- After clicking Save, you will see the tabulation result displayed in a matrix window.  By default, TransCAD shows he counts layer.  You can choose the other layers by going to the layers drop down list.

- Go to the toolbar and select Row Percent from the list of available layers.

- The cells in the matrix now sum up to 100% row-wise.

- The Row Percent view shows that men are more likely to undertake bicycle trips than women.

7

8

10

## Joint-Table Analysis

- Follow Steps 1-5 in Exercise 4.1 to open the Households.dbf file under /Survey.  Similarly, open the AllUrbanForm.dbf file under /UrbanForm.

- From the main menu, choose Dataview-Join.  TransCAD displays the Join dialog box.

- In the Join From area, set Persons table and HHID field.  In the To area, set Table to Households and Field to HHID.

- Click OK.  TransCAD will display the Persons+Households dataview.

- Now, go to Dataview-Join again.  This time, join the Persons+Households table and the AllUrbanForm table based on the HHID field.

- Click OK.  You will now see a Persons+Households+AllUrbanForm dataview table.

- You can now proceed with statistical analysis using all the fields in this joint table.

# Appendix I. Data Dictionary for the Households Dataset

| Variable | Type | Description |
| --- | --- | --- |
| hhid | Numeric | Household ID |
| zone | Numeric | MTC Regional TAZ Code |
| x_gcs | Numeric | Latitude |
| y_gcs | Numeric | Longitude |
| x_utm | Numeric | UTM x-coordinates |
| y_utm | Numeric | UTM y-coordinates |
| rcounty | Numeric | County id |
| famtype | Numeric | 5 category aggregated family type |
| famtype1 | Numeric | 10 category aggregated family type |
| hhsize | Numeric | Number in household |
| nkids | Numeric | Total number of kids |
| nkid04 | Numeric | Number of kids between the age of 0 to 4 |
| nkid510 | Numeric | Number of kids between the age of 5 to 10 |
| nkid1115 | Numeric | Number of kids between the age of 11 to 15 |
| nkid1617 | Numeric | Number of kids between the age of 16 to 17 |
| nactadul | Numeric | Number of active adults |
| nsenadul | Numeric | Number of senior adults |
| nemp | Numeric | Number of employed members |
| nprivp | Numeric | Number of privately employed members working for a pay |
| nprivnp | Numeric | Number of privately employed members without a pay |
| ngovt | Numeric | Number of members employed in the government |
| nself | Numeric | Number of self employed |
| nnopay | Numeric | Number of members working without pay |
| nftime | Numeric | Number of full-time employees |
| nptime | Numeric | Number of part-time employees |
| nmales | Numeric | Number of males |
| nfemales | Numeric | Number of females |
| nlic | Numeric | Number of licensed drivers |
| ndisable | Numeric | Number of individuals with disability |
| nftstud | Numeric | Number of full-time students |
| nptstud | Numeric | Number of part-time students |
| nnoflex | Numeric | Number of employees with inflexible work schedule |
| npflex | Numeric | Number of employees with partially flexible work schedule |
| nfflex | Numeric | Number of employees with fully flexible work schedule |
| hhveh | Numeric | Number of vehicles in household |
| hhbicyc | Numeric | Number of bicycles in household |
| hhmcycle | Numeric | Number of motorcycles/mopeds in household |
| hhincome | Numeric | 15 category aggregated household income |
| catinc | Numeric | 4 category aggregated household income |
| continc | Numeric | Continuous value of household income |
| tenure | Numeric | Variable indicating the tenure of household |
| user | Numeric | Dummy for users of non-motorized modes |
| type | Numeric | Variable indicating to which class does a HH belong |

### County ID (rcounty)

| Value | Description |
| --- | --- |
| 1 | San Francisco |
| 2 | San Mateo |
| 3 | Santa Clara |
| 4 | Alameda |
| 5 | Contra Costa |
| 6 | Solano |
| 7 | Napa |
| 8 | Sonoma |
| 9 | Marin |

### 5 category aggregated family type variable (famtype)

| Value | Description |
| --- | --- |
| 1 | Couple, two adults, married or unmarried |
| 2 | Nuclear Family, two adults and all kids <18 years |
| 3 | Single parent, one adult and all kids < 18 years |
| 4 | Single individual households, only one adult |
| 5 | Other family types |

### 10 category aggregated family type variable (famtype1)

| Value | Description |
| --- | --- |
| 1 | Couple |
| 2 | Nuclear Family, two adults and all kids <18 years |
| 3 | Young adult(s) with two parents with or without siblings |
| 4 | Single parent, one adult and all kids < 18 years |
| 5 | Young adult(s) with single parent with or without siblings |
| 6 | Single individual households, only one adult |
| 7 | Roommate households |
| 8 | Related individual households |
| 9 | Atleast one unrelated individual in household |
| 10 | Other family types |

### 15 category aggregated household income variable (hhincome)

| Value | Description |
| --- | --- |
| 1 | Below $10000 |
| 2 | $10000 to below $15000 |
| 3 | $15000 to below $20000 |
| 4 | $20000 to below $25000 |
| 5 | $25000 to below $30000 |
| 6 | $30000 to below $35000 |
| 7 | $35000 to below $40000 |
| 8 | $40000 to below $45000 |
| 9 | $45000 to below $50000 |
| 10 | $50000 to below $60000 |
| 11 | $60000 to below $75000 |
| 12 | $75000 to below $100000 |
| 13 | $100000 to below $125000 |
| 14 | $125000 to below $150000 |
| 15 | $150000 or more |
| 98 | Don’t know |
| 99 | Refused |

### 4 category aggregated household income variable (catinc)

| Value | Description |
| --- | --- |
| 1 | less than $50,000 |
| 2 | $50,000 to below $75,000 |
| 3 | $75,000 to below $125,000 |
| 4 | $125,000 or more |

### Tenure

| Value | Description |
| --- | --- |
| 1 | Rented house |
| 2 | Owned house |

### Type

| Value | Description |
| --- | --- |
| 0 | Other (did not travel) |
| 1 | Non-motorized households |
| 2 | Mixed households |
| 3 | Motorized households |

# Appendix II. Data Dictionary for the Persons Dataset

| Variable | Type | Description |
| --- | --- | --- |
| ID (personid) | Numeric | Key for person and HH(hhid*100+person) |
| hhid | Numeric | Household ID |
| person | Numeric | Person No. |
| female | Numeric | Gender |
| age | Numeric | Age of person |
| catage | Numeric | Categorized age |
| licdrive | Numeric | Variable indicating whether person is licensed to drive |
| disable | Numeric | Variable indicating whether person has a disability |
| ethncity | Numeric | 7 category aggregated ethnicity variable |
| ethnbg | Numeric | 5 category aggregated ethnicity variable |
| employed | Numeric | Variable indicating whether person works for pay |
| employer | Numeric | Variable indicating employer type |
| empstat | Numeric | Variable indicating employment status |
| numjobs | Numeric | Number of jobs |
| empfre | Numeric | Work frequency out of home |
| flexible | Numeric | Variable indicating flexibility of working hours |
| school | Numeric | Variable indicating whether person is a student |
| studstat | Numeric | Variable indicating student status |
| ninfo | Numeric | Variable indicating use of internet for browsing/information seeking |
| nshop | Numeric | Net use for browsing/shopping |
| nentgame | Numeric | Net use for entertainment and games |
| nsochat | Numeric | Net use for social purposes - e-mail/chat rooms |
| nbanfin | Numeric | Net use for banking and financial transactions/information |
| ntravel | Numeric | Net use for travel information and reservations |
| nstudy | Numeric | Net use for studying and/or on-line courses |
| ncdvideo | Numeric | Net use for CDs and Videos |
| motorper | Numeric | Number of motorized trips made by a person |
| bikeper | Numeric | Number of bicycle trips made by a person |
| walkper | Numeric | Number of walk trips made by a person |
| motorms | Numeric | Number of motorized trips for maintenance shopping trips |
| bikems | Numeric | Number of bicycle trips for maintenance shopping trips |
| walkms | Numeric | Number of walk trips for maintenance shopping trips |
| motorpj | Numeric | Number of motorized trips for prime job trips |
| bikepj | Numeric | Number of bicycle trips for prime job trips |
| walkpj | Numeric | Number of walk trips for prime job trips |
| motorpr | Numeric | Number of motorized trips for pure recreational trips |
| bikepr | Numeric | Number of bicycle trips for pure recreational trips |
| walkpr | Numeric | Number of walk trips for pure recreational trips |
| weekmot | Numeric | Number of motorized trips on weekday |
| weekbike | Numeric | Number of bicycle trips on weekday |
| weekwalk | Numeric | Number of walk trips on weekday |
| wkenmot | Numeric | Number of motorized trips on weekend |
| wkenbike | Numeric | Number of bicycle trips on weekend |
| wkenwalk | Numeric | Number of walk trips on weekend |
| user | Numeric | Dummy variable indicating that the person uses non-motorized modes |
| x_utm | Numeric | Home UTM x-coordinates |
| y_utm | Numeric | Home UTM y-coordinates |

### Age

| - Value | - Description |
| --- | --- |
| 997 | Less than 1 year |

### 6 category aggregated age variable (catage)

| - Value | - Description |
| --- | --- |
| - 1 | - Kid between 0-4 years |
| - 2 | - Kid between 5-10 years |
| - 3 | Kid between 11-15 |
| - 4 | Kid between 16-17 |
| - 5 | - Active adult |
| - 6 | - Senior adult |

### 7 category recoded ethnicity (ethnicity)

| Value | Description |
| --- | --- |
| 1 | Caucasian |
| 2 | African American |
| 3 | Hispanic |
| 4 | Asian/ Pacific Islander |
| 5 | European |
| 6 | Bi-racial, with one race being Caucasian |
| 7 | Other |

### 5 Category recoded ethnicity (ethnbg)

| Value | Description |
| --- | --- |
| 1 | Caucasian |
| 2 | African American |
| 3 | Hispanic |
| 4 | Asian/ Pacific Islander |
| 5 | Other |

### Variable indicating employer type (employer)

| Value | Description |
| --- | --- |
| 0 | Other |
| 1 | Privately employed working with pay |
| 2 | Privately employed but working without pay |
| 3 | Government employed |
| 4 | Self employed |
| 5 | Work for no pay |

### Employment status (empstat)

| Value | Description |
| --- | --- |
| 0 | Unemployed/Other |
| 1 | Full-time employee |
| 2 | Part-time employee |

### Flexibility of working hours (flexibility)

| Value | Description |
| --- | --- |
| - 0 | - Other |
| - 1 | - No flexibility |
| - 2 | - Partially flexible |
| - 3 | - Fully flexible |

### Student Status (studstat)

| Value | Description |
| --- | --- |
| 0 | Non-student/Other |
| 1 | Full time student |
| 2 | Part time student |

# Appendix III. Data Dictionary for the Trips Dataset

| Variable | Type | Description |
| --- | --- | --- |
| customid | Numeric | Key (hhid * 10000 + person * 1000 + day * 100 + activity) |
| personid | Numeric | Key for person and HH(hhid*100+person) |
| hhid | Numeric | Household ID |
| person | Numeric | Person ID |
| day | Numeric | Travel day |
| activity | Numeric | Activity number |
| startime | Numeric | Start time of trip |
| endtime | Numeric | End time of trip |
| duration | Numeric | Duration of trip |
| ttravdur | Numeric | Total travel time on trip (including change of transportation stops) to activity episode from previous activity episode |
| travdur | Numeric | Travel time on last leg to participate in an activity episode from previous episode or duration of pure recreation episode |
| trip | Numeric | Variable indicating trip purpose |
| mode | Numeric | Mode used on last leg to participate in an activity episode from previous episode |
| modenu | Numeric | 4 category aggregated mode variable |
| newmode | Numeric | 3 category aggregated mode variable |
| motor | Numeric | Motorized trips |
| bicycle | Numeric | Bicycle trips |
| walk | Numeric | Walk trips |
| changmod | Numeric | 1 if there is a change of mode during travel to the activity episode |
| stops | Numeric | Variable indicating number of changes of transportation stops during the trip |
| chaining | Numeric | Variable indicating number of stops during the trip |
| stopmins | Numeric | Stop minutes |
| fixeds | Numeric | Dummy variable indicating if stop timing was fixed |
| locnewpr | Numeric | Location where activity occurred |
| location | Numeric | General location of activity participation |
| locname | String | Name of location where activity occurred |
| loctype | String | Type of location where activity occurred (example church, bank, grocery, residential etc.) |
| locnum | Numeric | Location type number |
| loccity | String | City |
| locstate | String | State |
| loczip | Numeric | Zip |
| x_gcs | Numeric | Longitude of location |
| y_gcs | Numeric | Latitude of location |
| locgeo | String | Geographical result - TGEO |
| homeepis | Numeric | Indicator for home episode and relative location in individuals pattern |
| phyact | Numeric | Level of physical activity |
| physical | Numeric | Dummy variable for full physical activity |
| dayofwk | Numeric | Day of week of episode |
| weekend | Numeric | Dummy for weekend |
| month | Numeric | Month of travel |
| distim | Numeric | Discrete time of day |
| drivepas | Numeric | Variable indicating whether the individual was a driver or passenger during the trip |
| noinveh | Numeric | Number of individuals in the vehicle |
| uhhveh | Numeric | Dummy variable indicating whether vehicle from HH |
| notravel | Numeric | No travel before activity or pure recreation travel episode |

### Variable indicating trip purpose (trip)

| Value | Description |
| --- | --- |
| 0 | Other |
| 1 | Prime job trip |
| 2 | Maintenance shopping trip |
| 3 | Pure recreational trip |

### 4 categories trip purpose (trip)

| Value | Description |
| --- | --- |
| 0 | Other |
| 1 | Commute to primary job |
| 2 | Maintenance Shopping |
| 3 | Pure recreation |

### 4 category aggregated mode variable (modenu)

| Value | Description |
| --- | --- |
| 1 | Bicycle |
| 2 | Car/van/motorcycle/moped |
| 3 | Walk |
| 4 | Other |

### 3 category aggregated mode variable (newmode)

| Value | Description |
| --- | --- |
| 0 | Motorized |
| 1 | Bike |
| 2 | Walk |

### Variable indicating number of change of transportation stops during tip (stops)

| Value | Description |
| --- | --- |
| 0 | No change |
| 1 | One change of transportation stop |
| 2 | Two change of transportation stop |
| 3 | Three change of transportation stop |

### Variable indicating number of stops during trip (chaining)

| Value | Description |
| --- | --- |
| 0 | No stops |
| 1 | One stop sequence |
| 2 | Two stop sequence |
| 3 | Three stop sequence |

### Location where activity occurred (locnewpr)

| Value | Description |
| --- | --- |
| 1 | Home |
| 2 | Previously reported |
| 3 | New location |
| 4 | Don’t know |
| 5 | Refused |

### General location of activity participation (location)

| Value | Description |
| --- | --- |
| - 1 | - In or around home |
| - 2 | - Known out-of-home location |
| - 3 | - Is a trip to participate in an activity so location |
| - 4 | - Unknown location |
| - 5 | - Out-of-home location with unknown latitude/longitude |
| - 6 | - Is a pure recreation activity beginning and ending at home |

### Location type number (locnum)

| Value | Description |
| --- | --- |
| 0 | Not known |
| 1 | AA Meeting (Substance Abuse) |
| 2 | Alteration |
| 3 | Accountant |
| 4 | Acupuncture |
| 5 | Acoustical Engineering Firm |
| 6 | Activity Center |
| 7 | Ad/ Marketing Agency |
| 8 | Administration Office |
| 9 | Adoption agency |
| 10 | Adult day care / Nursing home |
| 11 | Adult entertainment |
| 12 | Aerospace engineering business/ firm |
| 13 | Aerobics |
| 14 | Military center/ base |
| 15 | Agriculture |
| 16 | Air freight/ cargo |
| 17 | Airport |
| 18 | Airline consulting |
| 19 | Airport parking |
| 20 | Alarm company |
| 21 | Amusement Park |
| 22 | Animal care (hospital/store) |
| 23 | Animation company |
| 24 | Antique shop |
| 25 | Apartments |
| 26 | Appliance store |
| 27 | Aquarium |
| 28 | Aquatics center |
| 29 | Architecture firm |
| 30 | Arcade |
| 31 | Archery |
| 32 | Arena |
| 33 | Army |
| 34 | Arts center/ studio |
| 35 | Art gallery |
| 36 | Art store |
| 37 | Assisted living |
| 38 | ATM |
| 39 | Attorney/ Law firm |
| 40 | Auction |
| 41 | Audio Production Company |
| 42 | Audiologist |
| 43 | Auditing firm |
| 44 | Auditorium |
| 45 | Auto dealer/ repair |
| 46 | Auto club |
| 47 | Auto insurance |
| 48 | Aviation |
| 49 | Bagel shop/ Bakery |
| 50 | Bail Bonds |
| 51 | Ball park |
| 52 | Ballet class |
| 53 | Bank |
| 54 | Banquet Hall |
| 55 | Bar/ Night club |
| 56 | Barber/ Beauty/ Nail Saloon |
| 57 | Barn/ Stable |
| 58 | BART station |
| 59 | Baseball card sales |
| 60 | Basketball arena |
| 61 | Batting cages |
| 62 | Beach |
| 63 | Bed and breakfast |
| 64 | Beverage company |
| 65 | Bicycle shop/ repair |
| 66 | Bike trail |
| 67 | Billiard hall |
| 68 | Billiard store |
| 69 | Bingo |
| 70 | Biotech company |
| 71 | Blood bank |
| 72 | Blueprinting |
| 73 | Boarding home |
| 74 | Boat Harbor |
| 75 | Boat repair store |
| 76 | Book publisher |
| 77 | Book store/ library |
| 78 | Bookkeeper |
| 79 | Bowling |
| 80 | Box office |
| 81 | Boy/ Girl scout camp/ center |
| 82 | Brewery |
| 87 | Bridal shop |
| 88 | Bridge |
| 89 | Bridge club |
| 90 | Brokerage firm |
| 91 | Builders |
| 92 | Building/ gardening |
| 93 | Bus stop/ station |
| 94 | Business/ commercial |
| 95 | Cabinet/ Countertop shop |
| 96 | Cable TV /services |
| 97 | Camera shop |
| 98 | Camp |
| 99 | Camping supply store |
| 100 | Cannery |
| 101 | Candle store |
| 102 | Candy store |
| 103 | Canoe rental |
| 104 | Carpool stop |
| 105 | Car rental |
| 106 | Carwash |
| 107 | Card shop |
| 108 | Career services |
| 109 | Carpentry |
| 110 | Carpet store |
| 111 | Catalog sales |
| 112 | Catering services |
|  | Cellular store |
|  | Cemetry |
|  | Ceramic studio/ store |
|  | Charity organization |
|  | Cheese factory |
|  | Chemical plant |
|  | Child care center |
|  | Children’s entertainment |
|  | Chiropractor |
|  | Chocolate mfg/ store |
|  | Christmas tree farm |
|  | Church |
|  | Cigarette store |
|  | Circus |
|  | City |
|  | Civic center |
|  | Civil Engineering |
|  | Clinic/ hospital |
|  | Clothing mfg/ store |
|  | Club |
|  | Coast guard |
|  | Coffee shop |
|  | Collectible shop |
|  | Collection agency |
|  | Coliseum |
|  | Communication business |
|  | Community center |
|  | Company |
|  | Computer industry |
|  | Computer consulting |
|  | Computer engineering |
|  | Computer repair/ sales |
|  | Computer software |
|  | Computer training |
|  | Concert hall |
|  | Concrete company |
|  | Conference center |
|  | Construction company |
|  | Construction site |
|  | Consultant |
|  | Consumer product company |
|  | Contractor |
|  | Convenience/ drug store |
|  | Convent |
|  | Convention center |
|  | Copy/ printing service |
|  | Cork company |
|  | Corner |
|  | Corporate office |
|  | Costume shop |
|  | Counseling |
|  | Country club |
|  | Craft store |
|  | Crane services |
|  | Credit card company |
|  | Credit counseling |
|  | Credit union |
|  | Cultural center |
|  | Customer service center |
|  | Dairy Products distribution center |
|  | Dairy ranch |
|  | Dam |
|  | Dance Concert/ Hall |
|  | Dance school/ club |
|  | Data Center |
|  | Data marketing systems |
|  | Day care |
|  | Defense |
|  | Deli |
|  | Delivery services |
|  | Dental office |
|  | Department store |
|  | Design Business |
|  | Development center |
|  | Diagnostics mfg |
|  | Diet center |
|  | Direct sales |
|  | Discount store |
|  | Distribution center |
|  | District attorney’s office |
|  | DK |
|  | Doctor |
|  | Donation Center |
|  | Downtown |
|  | Drilling company |
|  | Driving range |
|  | Driving school |
|  | Drug rehab |
|  | Drug manufacturing |
|  | Dry cleaner/ laundry |
|  | Dump |
|  | E-commerce |
|  | Education |
|  | Electric contract |
|  | Electricity production |
|  | Electronic industry |
|  | Electronics store |
|  | Elevator company |
|  | Employment service |
|  | Energy industry |
|  | Engineering |
|  | Engraving store |
|  | Entertainment |
|  | Environmental company |
|  | Equipment design87/ mfg |
|  | Errands |
|  | Event planning |
|  | Exhibit hall |
|  | Expo center |
|  | Export/ import sales |
|  | Exterminator |
|  | Eye care |
|  | Fabric store |
|  | Factory |
|  | Fair grounds |
|  | Family fun center |
|  | Farm/ ranch |
|  | Farmers market |
|  | Fax services |
|  | Ferry building/ dock |
|  | Festival/ parade |
|  | Fiber optics industry |
|  | Field |
|  | Field trip |
|  | Film lab |
|  | Film production |
|  | Film store |
|  | Financial services/ investing |
|  | Fitness class/ center |
|  | Fire protection company |
|  | Fire station |
|  | Fish store |
|  | Fishing |
|  | Flea market |
|  | Flight school |
|  | Flooring company/ contract |
|  | Flooring store |
|  | Florist |
|  | Flying club |
|  | Foam products store |
|  | Food bank |
|  | Food distribution center |
|  | Food manufacturing |
|  | Football stadium |
|  | Forest |
|  | Forestry Business |
|  | Forklift store/repair |
|  | Foundation |
|  | Frame shop |
|  | Freight company |
|  | Fraternity |
|  | Friend’s house |
|  | Fruit stand |
|  | Fund raising |
|  | Funeral home |
|  | Fur store |
|  | Furniture store |
|  | Gallery |
|  | Gambling |
|  | Game developing |
|  | Garage company |
|  | Garbage collection company |
|  | Garden hardware supply company |
|  | Gas company |
|  | Gas/ electric utility |
|  | Gas station |
|  | Geotechnical firm |
|  | Gift shop |
|  | Glass company |
|  | Golf club/ course |
|  | Golf supply store |
|  | Government/ Municipality/ City offices |
|  | Grandparents home |
|  | Graphic design firm |
|  | Grinding machine shop |
|  | Grocery |
|  | Gymnastic lessons |
|  | Hazardous waste |
|  | Health care |
|  | Heating/ air conditioning |
|  | Hi-tech industry |
|  | Historical place |
|  | Home improvement |
|  | Hotel/ Motel/ Inn |
|  | House |
|  | Housekeeping |
|  | Housing development |
|  | Human resources |
|  | Hunting club |
|  | Ice cream shop |
|  | Ice rink |
|  | Import business |
|  | Indoor rec/ sports |
|  | Industrial business |
|  | Internet |
|  | Insurance |
|  | Interior design |
|  | Intersection |
|  | Iron working |
|  | Janitorial services |
|  | Jewelry store |
|  | Junk yard |
|  | Karate/ martial arts classes |
|  | Labor organization |
|  | Lab research |
|  | Lake |
|  | Landscaping firm |
|  | Landfill |
|  | Laser industry |
|  | Learning center |
|  | Leasing center |
|  | Locksmith |
|  | Machine shop |
|  | Magazine shop |
|  | Mail services/ post office |
|  | Maintenance |
|  | Management services |
|  | Manufacturing |
|  | Marina |
|  | Meeting |
|  | Mortgage |
|  | Movie theatre |
|  | Movie rental |
|  | Moving company |
|  | Museum |
|  | Music hall |
|  | Music lessons/ school |
|  | Music store |
|  | Networking company |
|  | Non-profit org. |
|  | Nursery (plants) |
|  | Office |
|  | Office supply |
|  | Oil/ petroleum industry |
|  | Open space |
|  | Out of town/ state |
|  | Outdoors |
|  | Paint store |
|  | Park/ community garden |
|  | Parking (cars) |
|  | Party supplies |
|  | Payroll services |
|  | Pension services |
|  | Pharmacy |
|  | Phone call |
|  | Phone company/ store |
|  | Pick-up someone |
|  | Plastic industry |
|  | Plaza |
|  | Pool/swim center |
|  | Private property |
|  | Production |
|  | Professional services |
|  | Property management |
|  | Public relations |
|  | Public utility company |
|  | Publisher |
|  | Race track |
|  | Radio station |
|  | Railroad station |
|  | Real estate |
|  | Recording company |
|  | Recreation |
|  | Recruiting |
|  | Rental store |
|  | Repair business |
|  | Research |
|  | Reservoir |
|  | Resort/ spa |
|  | Residential |
|  | Rest area |
|  | Restaurant |
|  | Retail |
|  | Running/walking |
|  | RV sales/ repair |
|  | Satellite company |
|  | School |
|  | School supply |
|  | School district offices |
|  | Science industry |
|  | Security offices |
|  | Senior community |
|  | Shelter |
|  | Shopping |
|  | Sign company |
|  | Skating/ skiing |
|  | Soccer |
|  | Social activity |
|  | Sporting event |
|  | Sporting / outdoor goods |
|  | Storage |
|  | Steel/ Building material |
|  | Street |
|  | Swimming lessons |
|  | Swimming pool supply store |
|  | Tax service |
|  | Taxi |
|  | Technology industry |
|  | Telecommunication |
|  | Tennis |
|  | Testing |
|  | Theatre |
|  | Therapy |
|  | Title company |
|  | Tour/ Tourist attraction |
|  | Tow trucking |
|  | Trading |
|  | Training company |
|  | Transit agency/company |
|  | Transportation stop |
|  | Travel agency |
|  | Trucking company |
|  | Utilities company |
|  | Veterans hall |
|  | Vineyard |
|  | Volunteer work |
|  | Voting |
|  | Warehouse |
|  | Waste management |
|  | Wedding |
|  | Welding |
|  | Wholesale industry |
| 435 | Wireless communication |
| 436 | Wood working |
| 437 | Work |
| 438 | Yacht |
| 439 | YMCA/ youth club |
| 440 | Zoo |
| 441 | Plumbing services |
| 442 | Plumbing supply |

### Indicator for home episode and relative location in individuals pattern (homeepis)

| - Value | - Description |
| --- | --- |
| - 1 | - Episode is the first episode of the day |
| - 2 | - Episode is preceded y a return home trip |
| - 3 | - Episode preceded by another home episode |
| - 4 | - Does not apply because it is a trip episode |
| - 5 | - Does not apply because it is an out-of-home episode |
| - 6 | - Does not apply because is a stop or change of mode episode |

### Level of physical activity (phyact)

| - Value | - Description |
| --- | --- |
| - 0 | - No physical activity or not a recreational activity |
| - 1 | - Fully physical activity (gym/fitness classes etc.) |
| - 2 | - Partially physical activity (being outdoors, going to the beach) |

### Day of the week of episode (dayofwk)

| Value | Description |
| --- | --- |
| 1 | Sunday |
| 2 | Monday |
| 3 | Tuesday |
| 4 | Wednesday |
| 5 | Thursday |
| 6 | Friday |
| 7 | Saturday |

### Dummy for weekend (weekend)

| - Value | - Description |
| --- | --- |
| 0 | Weekday |
| 1 | Weekend |

### Month of travel (month)

| Value | Description |
| --- | --- |
| 1 | January |
| 2 | February |
| 3 | March |
| 4 | April |
| 5 | May |
| 6 | June |
| 7 | July |
| 8 | August |
| 9 | September |
| 10 | October |
| 11 | November |
| 12 | December |

### Discrete time of day (distim)

| - Value | - Description |
| --- | --- |
| 1 | 3:01 to 4 am |
| 2 | 4:01 to 5 am |
| 3 | 5:01 to 6 am |
| 4 | 6:01 to 7 am |
| 5 | 7:01 to 8 am |
| 6 | 8:01 to 9 am |
| 7 | 9:01 to 10 am |
| 8 | 10:01 to 11 am |
| 9 | 11:01 to 12 pm |
| 10 | 12:01 to 1 pm |
| 11 | 1:01 to 2 pm |
| 12 | 2:01 to 3 pm |
| 13 | 3:01 to 4 pm |
| 14 | 4:01 to 5 pm |
| 15 | 5:01 to 6 pm |
| 16 | 6:01 to 7 pm |
| 17 | 7:01 to 8 pm |
| 18 | 8:01 to 9 pm |
| 19 | 9:01 to 10 pm |
| 20 | 10:01 to 11 pm |
| 21 | 11:01 to 12 am |
| 22 | 12:01 to 1 am |
| 23 | 1:01 to 2 am |
| 24 | 2:01 to 3 am |