Lab 3b—Continuity Equation

Name____________________________________Date____________Time___________

Data

Container: Circular with D=______m			      At=________m2

Hole: Circular with  d =______m			                 Ah=________m2

Assume vh= (2gh)1/2	                 From Bernoulli equation (to be  covered soon in class)

h0=________m

hf =________m

Time readings:

With the lid on: t1= ______________________ (s)

Without the lid: t2=_______________________ (s)

a) Estimate how long it would take for the water level to go from h0 to hf when the container doesn’t have the lid on.

Using the continuity equation, we get (see the lab description for details):

v2Ah=−Atdhdt		Substituting v2=2gh and integrating we obtain:

t=−2At2gAhh12+C1			When t=0 h=h0:	C1=2At2gAhh012

t=+2At2gAhh012−hf12

t=   − =_______s

Relative Error:  t−t2t=_____________

Explain why t is smaller/larger than t2

b) When the container has the lid on, it requires more/less time (circle the answer). Explain why

Determine when the water in the tank will stop running out of the hole

Given

H=________cm

ho=________cm

ρ=ρH20 = ________kg/m3

g=________m/s2

Patm = _________   N/m2

- Hydrostatic law applied since v2=0 :   P1=P2−ρgh → P1=Patm−ρgh        (1)

- Ideal gas law:  PV=nRT=const       thus P1(H−h)At=Patm(H−ho)At → P1=PatmH−hoH−h                              (2)

From (1) and (2), we have

PatmH−hoH−h=Patm−ρgh or −ρgh=PatmH−hoH−h−1 → ρgPatm=1hh−h0H−h=h0−hhH−h2

C2=ρgPatm=________________Thus C2=ho−hhH−h2 → hHC2− C2h2=ho−h h2−h(H+1C2)+hoC2=0 (∗)

Solve Equation (*), we have

Δ=(H+1C2)2−4h0C2=________________________

h=(H+1C2)±Δ2=________________________  (m)

Compare h with h0 ________________________

Time to reach v=0

At t=t0= 0   v0=2gho=____________________ (m/s)

At t=t          vt = 0 m/s

Assuming v changes linearly with t, we have

v=v0+vt2=_______________________ (m/s)

Thus  Δt=Δhv=ho−hv= __________________ (s)