A Comparison of the Four-Step Versus Tour-Based Models in the Context of Predicting Travel Behavior Before and After Transportation System Changes

| Nazneen Ferdous Resource Systems Group, Inc. 55 Railroad Row White River Junction, VT 05001 Tel: (802) 295-4999, Ext: 118 ; Fax: (802) 295-1006 Email: Nazneen.Ferdous@rsginc.com | Chandra R. Bhat (corresponding author) The University of Texas at Austin Dept of Civil, Architectural & Environmental Engr 1 University Station C1761, Austin TX 78712 Tel: 512-471-4535; Fax: 512-475-8744 Email: bhat@mail.utexas.edu |
| --- | --- |
| Lakshmi Vana London Business School Regents Park London NW1 4SA. UK Tel: +44 07501209686 Email: v.l.n.prasad@gmail.com | David Schmitt AECOM 300 East Broad Street, Suite 300 Columbus, OH 43215 Tel: 614-429-5094; Fax: 614-429-5101 Email: david.schmitt@aecom.com |
| John L. Bowman Bowman Research and Consulting 28 Beals St, Brookline, MA 02446 Tel: 617-232-8189 Email: john_L_Bowman@alum.mit.edu | Mark Bradley Mark Bradley Research and Consulting 524 Arroyo Ave., Santa Barbara, CA 93109 Tel: 805-564-3908 Email: mark_bradley@cox.net |
| - Ram M. Pendyala Arizona State University School of Sustainable Engr and the Built Environment Room ECG252, Tempe, AZ 85287-5306 Tel: 480-727-9164; Fax: 480-965-0557 Email: ram.pendyala@asu.edu | Rebekah Anderson Ohio Department of Transportation 1980 West Broad Street, Columbus OH, 43223 Tel: 614-752-5735; Fax: 614-752-8646 Email: rebekah.anderson@dot.state.oh.us |
| - Gregory Giaimo Ohio Department of Transportation 1980 West Broad Street, Columbus OH, 43223 Tel: 614-752-5738; Fax: 614-752-8646 - Email: greg.giaimo@dot.state.oh.us |  |

Abstract

The main objective of this study is to examine the performance of the MORPC trip-based and tour-based frameworks in the context of three specific projects started and completed within the past 20 years in the Columbus metropolitan area. Regional- and project-level comparisons of the performance of the trip-based and tour-based models are made for three scenario years: 1990, 2000 and 2005. The regional-level analysis is undertaken in the context of four travel dimensions based on data availability and observed data to model output compatibility. These four dimensions are vehicle ownership, work flow distributions, work flow distribution by time-of-day, and average work trip travel times. The tour-based model performs better overall than the trip-based model for all these four dimensions. The project-level comparative assessment of the predicted link volumes from the trip-based and the tour-based models is undertaken with respect to the observed link counts and by roadway functional class. The results did not show any clear trends in terms of performance of the models by functional class or year.

1. Introduction

The need to examine individual-level behavioral responses, and accurately forecast long-term travel demand in a rapidly changing demographic context, has led to a behaviorally-oriented tour-based approach to travel demand modeling. Indeed, the potential benefits of the tour-based approach, combined with the increasing levels of demands placed by legislations on the abilities of travel demand models, has led several planning agencies in the United States to shift (or consider the shift) to the tour-based approach. The Mid-Ohio Regional Planning Commission (MORPC) is one of the agencies that adopted a fully operational tour-based model, for the Columbus region.  Subsequently, the Ohio Department of Transportation (ODOT) developed a parallel traditional trip-based model from the same data as used for the tour-based model for use in a research study.  This presence of both a trip-based and a fully operational tour-based model provides a unique opportunity to test and compare the models for their policy sensitivity and forecasting ability. Accordingly, the main objective of this paper is to examine and compare the performance of the MORPC trip-based and tour-based frameworks in the context of specific highway projects. Toward this end, the current paper presents an analysis and assessment of the accuracy of predicted travel patterns by the trip-based and the tour-based models of MORPC before and after several highway projects.

The rest of the paper is organized as follows. Section 2 discusses study projects and a control area identified for the analysis. Data preparation tasks undertaken for the study projects are briefly discussed in Section 3. Empirical comparison exercises between model outputs and observed data are presented in Section 4. The final section concludes the paper by summarizing important findings and recommendations.

2. Study Projects and Control Area

The emphasis of the current research study is to compare predictions of travel behavior before and after major developments and roadway projects that have started and been completed in the past 15 years or so in the Columbus metropolitan area. Accordingly, the following projects and control area were identified for undertaking before-and-after effects analysis (see Figure 1 for the geographic locations of the selected projects and the control area):

Polaris: The Polaris region has seen large retail and employment growth in the last 20 years. The roadway improvements that coincide with this land-use growth include: (1) I-71 interchange with Polaris Parkway and new Polaris Parkway completed in 1993, (2) Polaris parkway widening completed in early 2000, and (3) I-71 split interchanges with Polaris Parkway and Gemini Parkway completed in 2007.

Hilliard-Rome project: No major roadway improvements were undertaken in this study area between 1990 and 2005. However, the Hilliard-Rome Road and the region on the west side of Columbus around I-70/I-270 have experienced large land-use related developmental changes in the late 1990s and early 2000s.

Spring-Sandusky interchange project: The Spring-Sandusky interchange project involved (1) reconstruction of SR 315 between I-670 and I-70/71, (2) new construction of the portion of I-670 between I-70 and SR 315 and (3) reconstruction and widening of I-670 between SR 315 and I-71. The project did not directly attract any substantial land use related changes. The project started in 1993 and was completed in 2003.

Control area: A control area with no substantial land use and network changes to significantly alter travel patterns in the time period under consideration is identified (the time period under consideration is 1990 to 2005; the years 1990, 2000, and 2005 are the three analysis years used in the current analysis, as discussed further in Section 4). The selected control area is I-71 bounded by Harrisburg Pike (SR 3) and I-270 in southern Franklin County.

3. Data Preparation Efforts for Study Areas

A study area was established for each project to reflect the geographic location within which roadway link volumes would most substantially be impacted directly from the planned developments. A detailed review of the roadways was undertaken for each study area, including verifying the accuracy of roadway connectivity, lane configuration, and traffic counts. Both the trip-based and tour-based models used identical highway networks for each analysis year.

Demographic data were generated for both models for each of the three years (1990, 2000, and 2005) based on Census data (see Ferdous et al. (11) for more details). Some variables were added to the trip-based model dataset to reflect the travel generation needs for that model. Income is represented in year 2000 dollars in all analysis years.

Six model runs were developed: one for each analysis year (1990, 2000, and 2005) and each model (trip-based and tour-based). The trip-based model runs one iteration of feedback to mode choice with no convergence criteria. The tour-based model runs two iterations of feedback to travel generation with no convergence criteria. Both models use the identical equilibrium highway assignment closure criteria during the initial highway assignment(s) (a relative gap of 10-3 or 200 iterations, whichever is reached first). For the final highway assignment procedures, 500 iterations of equilibrium were specified.

After each model run, post-processing scripts were applied to the output files to generate the datasets used in the current study. The post-processing scripts varied slightly for each model to account for the different units of travel and trip purposes.

4. Empirical Comparison Exercise

This section discusses the performance of the MORPC trip-based and tour-based models. The performance evaluation of the models is pursued at two levels. The first level corresponds to a region-level analysis (independent of the specific project identified in Section 2) that compares selected model outputs from each of the trip-based and tour-based model systems with corresponding region-level observed data. The second level corresponds to a local-level analysis (specific to each of the three projects and the control area identified in Section 2) that compares the model predicted link volume outputs on selected roadways in and around the project region with corresponding observed link counts. For both the region-level and local-level analysis, we consider three years for analysis, as identified below:

- Model year 1990: This is the base year/ no-build case; construction of the selected study projects did not begin prior to this year.

- Model year 2000: The Hilliard-Rome project was complete, the Polaris Interchange (Phases 1 and 2 of 3) was complete, while the Spring-Sandusky Interchange was under construction.

- Model year 2005: The Hilliard-Rome project, Spring-Sandusky Interchange, and the first two phases of the Polaris project were complete, while Phase 3 of the Polaris project was not yet constructed.

The fit measures employed for comparison of model attributes with the observed data (for both the region-level and local-level analyses) are the Absolute Percentage Error (APE) measure and the Root Squared Error (RSE) measure, defined as follows:

APE=|(Observed Data−Predicted Data)|Observed Data×100

RSE=(Observed Data−Predicted Data)2

The measures above were computed for each “cell”, where a cell represents an appropriate spatial context in each of the region-level and local-level analyses (for example, a “cell” may be a specific county-to-county work flow). We also developed a weighted mean of the absolute percentage error statistic that was computed as the sum of the absolute percentage error for each cell weighted by the fraction of observations in that cell.  Similarly, we computed a root weighted mean square error as the root of the sum of the squared error for each cell weighted by the fraction of observations in that cell. The results of the comparison exercise allow us to understand the relative predictive capabilities of the trip-based and tour-based model frameworks. In the subsequent sections, we present comparative performance assessment of the trip-based and the tour-based models with the observed data.

4.1 Region-Level Comparison

A number of data sources were used to undertake the comparison between the model outputs and the observed data. These included, for the most part, the Census Summary Files 3 (SF3) (for the years 1990 and 2000), the 1999 Household Interview Survey (HIS) (for the year 2000), and the American Community Survey (ACS) (for the year 2005). In the rest of this paper, we will refer to the Census SF3 data simply as the Census data.

The geographic coverage of the HIS matches up with the MORPC study region that includes Delaware, Franklin, and Licking counties completely and Fairfield, Madison, Pickaway, and Union counties partially. However, the Census and the ACS data correspond to entire counties in the region. As a result, the comparisons between the HIS data and the trip/tour-based model are one-to-one from a spatial coverage standpoint, while the comparisons between the Census/ACS data and the trip/tour-based model for Fairfield, Madison, Pickaway, and Union counties (these are the counties represented only partially in the study region) need to be interpreted with caution. For these counties that are only partially contained in the study region, the travel quantities (such as car ownership levels and total work flows in and out of counties) as obtained from the Census and ACS data are factored down based on the percentage area of the county in the study region relative to the total area of the county.  (Alternative factoring methods, such as those based on number of county households in the study region relative to total county households in the county, county population in the study region relative to total county population, and number of county workers in the study region relative to total workers in the county, were also considered, but these alternative methods provided similar results.)

The model attributes evaluated in this section include household vehicle ownership level, county level O-D work flow distribution, split in work trip start time distribution by time of day (peak and off-peak period) and county of residence, and average travel time for work trips by county of residence. The results corresponding to these model attributes are presented and discussed in the subsequent sections.

4.1.1 Vehicle Ownership

Table 1a presents the results for vehicle ownership level by county for the year 1990. Similarly, Tables 1b and 1c show the results of the performance metrics of the trip-based and tour-based models in comparison to the 2000 Census and the 1999 Household Interview Survey (HIS), respectively, and Table 1d presents the results for the year 2005 compared to the 2005 American Community Survey (ACS). Several interesting observations may be made from Tables 1a through 1d. Across all years, the tour-based model outperforms the trip-based model in terms of vehicle ownership model predictions for Franklin County. This is important, because Franklin County represents about 80% of the population of households and overall activity-level in the study region. Given that vehicle ownership impacts several other activity-travel decisions downstream in the modeling framework, and the vehicle ownership prediction for a substantial fraction of the study region is better from the tour-based model, it may be expected that the tour-based model would provide better disaggregate-level predictions for specific activity-travel dimensions and may better be able to examine policy response effects. Interestingly, the trip-based model predictions of vehicle ownership are superior to the tour-based model predictions for essentially all non-Franklin counties and for all years and all data sets. This consistent underperformance of the tour-based model for non-Franklin counties is an issue that needs to be tagged for further examination in future model development efforts. Overall, across the entire study region, the tour-based model performs somewhat better than the trip-based model in 1990 and 2000 when compared with the Census data, while the trip-based model performs somewhat better than the tour-based model in 2000 (compared to the HIS data) and in 2005 (compared to the ACS data). It is also interesting to note that the error measures are about the same magnitude across the many years, suggesting that the vehicle ownership components of the trip-based and tour-based models perform reasonably well when temporally transferred to other years.

4.1.2 Work Flow Distributions

Tables 2a through 2d present performance measures for person work flow distributions in a county-level origin-destination format. For Tables 2a, 2b, and 2d, the trip-based and the tour-based model outputs are compared with the observed person work flows from each county to within that county and to outside that county. This was because flow information was available only at this level from the Census SF3 data and the ACS data. However, for Table 2c, the models are compared with the observed county-to-county person work flows, since county-to-county work flows are available from the 1999 HIS.

The results in Tables 2a through 2d indicate that, in general, the tour-based model performs better than the trip-based model. This is particularly so for inter-county flows, as can be observed from the final row entitled “Total flows/overall weighted mean error” for the column entitled “outside origin county” in Tables 2a, 2b, and 2d (for comparison with the 1990 Census, the 2000 Census, and the 2005 ACS, respectively). Specifically, the overall weighted mean error measures for the tour-based model are consistently lower for the “outside origin county” flows than the corresponding flows from the trip-based model. In particular, the flows originating in Delaware, Franklin, and Licking counties (the three largest counties in the study area in terms of work trip generation) and destined outside these counties are better predicted by the tour-based model for all years (i.e., 1990, 2000, and 2005). For work flows originating from the remaining counties (Fairfield, Madison, Pickaway, and Union) and terminating outside these counties, the tour-based model provides somewhat better results in 1990 and the trip-based model provides clearly better results for 2000 and 2005. For intra-county flows, both the trip-based and tour-based models provide about the same results for Franklin and Licking counties (the largest two counties in terms of work trip generation), while the trip-based model clearly performs better for Delaware and Fairfield counties.  The trip-based model also performs better in 2000 for Madison and Pickaway counties, while the tour-based model is superior for Union county in that year. The comparison with the HIS data in Table 2c again indicates the better overall performance of the tour-based model for work flows originating from Franklin County (the largest county in terms of work flow), though the trip-based model performs better for work flows from Licking County (especially, the work flow from Licking to Franklin County). But, overall, even from the HIS data comparison, the tour-based model performs better than the trip-based model for county-to-county work flows, as can be observed from the final row of Table 2c.

4.1.3 Work Flow Distribution by Time-of-Day of Trip Start

Table 3 presents the error statistics for the work flow distribution by county of origin and two times of the day of the work trip start from the home end: the peak period (6:30 am to 9:29 am and 3:30 pm to 6:29 pm) and the off-peak period (all times that do not fall within the peak period). The results consistently and across years show the tour-based model to be a better match overall of the observed peak period and off-peak period work flow distributions compared to the trip-based model (see the final row labeled “Overall weighted mean error”). This is not surprising, given that the tour-based model consistently outperforms the trip-based model predictions of work flow distribution by time of day of trip start for work trips originating in Franklin County (which is the largest generator of work trips). The tour-based model also does better for work trip flow distribution by trip start time for trips from Fairfield County (except for the off-peak period in 1990). Interestingly, though, for Licking County (the second largest generator of work trips), the trip-based model performs better than the tour model in 1990 and 2000, but not in 2005.

In summary, the work flow distribution by time of day forecasting ability of the tour-based model is better in the overall than the trip-based model for all years based on the Census/ACS data.

4.1.4 Average (Person) Work Trip Travel Time

The average (person) work trip travel times from the trip-based and tour-based models are compared next with the corresponding values from the Census and ACS data sets (see Table 4). The results indicate that, except for Fairfield County for the year 1990, the average work trip travel time predictions for other counties and all years from the tour-based model are better than or about the same or only marginally worse than from the trip-based model. This generally better performance of the tour-based model is also clear from the overall weighted (by flow from origin county) mean error rows of the table, indicating an edge for the tour-based model over the trip-based model (across all years) in terms of average work trip travel time prediction.

4.2 Project-Level Comparison

This section presents a comparative assessment of the predicted link volumes from the trip-based and the tour-based models with the observed link counts. The observed link counts were available only at an annual average daily traffic (AADT) level. The fit measures employed for comparison of model predicted link volumes with the observed counts are the Absolute Percentage Error (the APE error measure was also used in the region-level comparison) and the Percentage Root Mean Squared Error (%RMSE), defined as follows:

%RMSE=i=1N(Observed Counti−Predicted Counti)2N−1i=1NObserved CountiN×100

where i is an index for road link (i = 1, 2, …, N). We also calculated a weighted mean of the %RMSE statistic that was computed as the sum of the percentage root mean squared error for each cell weighted by the fraction of observations in that cell.

Table 5 presents the observed link volumes and the model results for each study project and model year, aggregated by roadway functional class. For the Polaris project, the tour-based model provides clearly better results than the trip-based model for 1990, while the two models perform about equally well for 2000 and 2005 (based on the overall weighted mean across all links in the Polaris study area). Across all years, the tour-based model provides better predictions for the freeway functional class. However, the trip-based model is better than the tour-based model in terms of predicting flows on major arterial links.

For the Hilliard-Rome project, the tour-based model provides better results (relative to the trip-based model) for the freeway functional class in 1990 and 2005, but worse results (relative to the trip-based model) for the freeway functional class in 2000. The tour-based model also provides better results for the major arterial class in 2000 and 2005.

For the Spring-Sandusky project, the tour-based model provides worse results for the freeway and expressway functional class, but performs marginally better for the major arterial class. Overall, the predictive power of the tour-based model is marginally lower than from the trip-based model.

Finally, for the control area, the tour-based model predicts link flows on freeways with a better accuracy than the trip-based model for 1990, though the roles are reversed for 2005. There is no difference in predictive ability for the freeway functional class in 2000. The trip-based model’s performance is also superior to that of the tour-based model for the major arterial class.

Overall, the results from the trip-based and tour-based models indicate about equal predictive abilities for both the before-project and after-project situations at the level of link predictions (see final three rows of Table 5). It is difficult to make a strong case for one of the MORPC models being superior to the other from this standpoint. It should be noted that the use of a traditional static traffic assignment process does, to an extent, “undo” the benefits of the fine resolution of time represented in the tour model. This happens because the tours are grouped back to four aggregate time periods in the assignment stage and the static assignment process does not consider the dynamics of vehicle delays (see also Pinjari et al. (6)). In general, the results in this section do provide validation that the tour-based model, being a more recent entrant to the travel demand practitioner’s toolbox, is producing reasonable results at the link level.

5. Conclusions

This study compared the performance of the MORPC trip-based and the tour-based models with regional-level information from the Census, ACS, and HIS as well as project-level information before and after projects. Such a comparative exercise provides a good opportunity for both models to be tested for their travel behavior and forecasting ability.

Regional- and project-level comparisons were made for three scenario years: 1990, 2000 and 2005. The tour-based model performed slightly better overall than the trip-based model in the regional-level comparisons. It performed better than the trip-based model, with some exceptions, in terms of vehicle ownership levels, work flow distribution, work start time distribution, and the average travel time for work trips. Neither model distinguished itself in the project-level comparison of link flows, as both models generally produced the same level of accuracy.

Through this analysis, the project team has learned firsthand the difficulties of making disaggregate model comparisons when the models have different units of travel. A major challenge is that translating the results to a common unit of travel generally causes inconsistencies except when performed at an aggregate level, because one must apply off-model rules to convert one model’s data set to the other model’s unit of travel.  Given this challenge, definitive statements about the superiority of one model over the other are not easily made. Generally, the performance of the tour-based model in these specific tests provides evidence of the ability of these types of models to provide decision makers with better information on travel behavior. The MORPC tour-based model’s vehicle ownership procedures appear to need further investigation, as they underperformed in all counties except for Franklin County.

The performance of the tour-based model in the project situations was somewhat disappointing, even if it performed about as well as the trip-based model. The results suggest that this tour-based model will not forecast better than traditional methods without additional behavioral resolution, network resolution, validation procedures or some combination thereof. It should, however, also be pointed out that the study projects selected in this analysis corresponded to land-use developments and roadway supply enhancements, not to demand-management actions. There is a need in the future to examine the performance of the trip- and tour-based models in the context of demand-management strategies.

This research effort is an important step toward a better understanding of the tangible benefits of disaggregate tour-based modeling methods. But it should be viewed as only one step. It would be imprudent to judge all model systems strictly on the results of this one project, since the transportation planning community has accumulated four decades of learning and experience on trip-based models while this particular tour-based model represents only one attempt, and one of the earliest, at implementing the tour-based or activity-based approach for practical use. Regardless, the results in this paper should serve as an important reference in the assessment of the potential practical benefits of disaggregate tour-based modeling approaches vis-à-vis aggregate trip-based methods.

Acknowledgments

The authors acknowledge the helpful comments of four anonymous reviewers on an earlier version of the paper. This paper was a result of a larger study undertaken for the Ohio Department of Transportation (ODOT). Funding from ODOT is gratefully acknowledged. Lisa Macias helped with formatting this document.

REFERENCES

- Bowman, J.L., and M.A. Bradley. Activity-Based Travel Forecasting Model for SACOG: Technical Memos Numbers 1-11, 2005-2006. Available at http://jbowman.net.

- Pinjari, A.R., and C.R. Bhat. Activity-Based Travel Demand Analysis. In A Handbook of Transport Economics, A. de Palma, R. Lindsey, E. Quinet, and R. Vickerman (eds.), Ch. 6, pp. 213-248, Edward Elgar Publishing Ltd, Cheltenham, U.K., 2011.

- Miller, E.J., and P.A. Salvini. The Integrated Land Use, Transportation, Environment (ILUTE) Microsimulation Modelling System: Description & Current Status. Presented at the 9th Meeting of the International Association of Travel Behavior Research, Queensland, Australia, 2000.

- Bhat, C.R., J.Y. Guo, S. Srinivasan, and A. Sivakumar. Comprehensive Econometric Microsimulator for Daily Activity-Travel Patterns. In Transportation Research Record: Journal of the Transportation Research Board, No. 1894, Transportation Research Board of the National Academies, Washington, D.C., 2004, pp. 57-66.

- Bhat, C.R., J. Guo, S. Srinivasan, A. Pinjari, N. Eluru, R. Copperman, and I.N. Sener. The Comprehensive Econometric Microsimulator for Daily Activity-Travel Patterns (CEMDAP). Report 4080-S, prepared for the Texas Department of Transportation, Center for Transportation Research, The University of Texas at Austin, 2006.

- Pinjari, A., N. Eluru, R. Copperman, I.N. Sener, J.Y. Guo, S. Srinivasan, and C.R. Bhat. Activity-Based Travel-Demand Analysis for Metropolitan Areas in Texas: CEMDAP Models, Framework, Software Architecture and Application Results. Report 4080-8, prepared for the Texas Department of Transportation, Center for Transportation Research, The University of Texas at Austin, 2006.

- Pendyala, R.M., R. Kitamura, A. Kikuchi, T. Yamamoto, and S. Fujii. FAMOS: The Florida Activity Mobility Simulator. In Transportation Research Record: Journal of the Transportation Research Board, No. 1921, Transportation Research Board of the National Academies, Washington, D.C., 2005, pp. 123-130.

- Pendyala, R.M. Phased Implementation of a Multimodal Activity-Based Travel Demand Modeling System in Florida. Final Report, Volume II: FAMOS Users Guide. Research Center, Florida Department of Transportation, Tallahassee, FL, 2004.

- Goulias, K.G., C.R. Bhat, R.M. Pendyala, Y. Chen, R. Paleti, K.C. Konduri, T. Lei, D. Tang, S.Y. Youn, G. Huang, and H.H. Hu. Simulator of Activities, Greenhouse Emissions, Networks, and Travel (SimAGENT) in Southern California. Presented at the 91st Annual Meeting of the Transportation Research Board, Washington, D.C., January 2012.

- Arentze, T. and H. Timmermans. ALBATROSS: A Learning Based Transportation Oriented Simulation System. Technische Universiteit Eindhoven, Netherlands, 2001.

- Ferdous, N., C. Bhat, L. Vana, D. Schmitt, J.L. Bowman, M. Bradley, and R. Pendyala. Comparison of Four-Step versus Tour-Based Models in Predicting Travel Behavior Before and After Transportation System Changes – Results Interpretation and Recommendations. Report number FHWA/OH-2011/4, Ohio Department of Transportation, Office of Research and Development, 2011. Available at:

http://www.dot.state.oh.us/Divisions/Planning/SPR/Research/reportsandplans/Reports/2011/Planning/134368_FR.pdf.

LIST OF FIGURE AND TABLES

Figure 1	Selected Study Projects and Control Area

Table 1a 	Vehicle Ownership Level by County – Comparison with the Census Data (Year 1990)

Table 1b 	Vehicle Ownership Level by County – Comparison with the Census Data (Year 2000)

Table 1c 	Vehicle Ownership Level by County – Comparison with the HIS Data (Year 2000)

Table 1d 	Vehicle Ownership Level by County – Comparison with the ACS Data (Year 2005)

Table 2a 	Work Flow Distribution by County – Comparison with the Census Data (Year 1990)

Table 2b 	Work Flow Distribution by County – Comparison with the Census Data (Year 2000)

Table 2c 	Work Trip Flow Distribution by County – Comparison with the HIS Data (Year 2000)

Table 2d 	Work Flow Distribution by County – Comparison with the ACS Data (Year 2005)

Table 3 	Work Flow Distribution by Trip Start Time – Comparison with the Census/ACS Data

Table 4 	Travel Time for Work Trips – Comparison with the Census Data (Year 1990)

Table 5 	Project Level Link Volume Comparison by Roadway Functional Class

Polaris Study Area

Hilliard-Rome Study Area

Spring-Sandusky Study Area

Control Area

FIGURE 1 Selected study projects and control area.

TABLE 1a Vehicle Ownership Level by County – Comparison with the Census Data (Year 1990)

| County | Vehicle ownership level (VOL) | Number of households in vehicle ownership level from Census | Absolute Percentage Error (APE) |  |  |  | Root Squared Error (RSE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip-based model |  | Tour-based model |  | Trip-based model |  | Tour-based model |  |
|  |  |  | By VOL | Wtd. Mean | By VOL | Wtd.Mean | By VOL | Wtd.Mean | By VOL | Wtd.Mean |
| Delaware | No vehicle | 918 | 54.36 | 19.07 | 98.79 | 50.65 | 499 | 1,606 | 907 | 3,796 |
|  | 1 vehicle | 5,363 | 4.34 |  | 70.70 |  | 233 |  | 3,792 |  |
|  | 2 vehicles | 10,525 | 18.65 |  | 43.12 |  | 1,963 |  | 4,538 |  |
|  | 3+ vehicles | 6,310 | 27.16 |  | 39.15 |  | 1,714 |  | 2,470 |  |
| Fairfield | No vehicle | 805 | 32.56 | 31.80 | 10.05 | 34.07 | 262 | 1,425 | 81 | 2,308 |
|  | 1 vehicle | 3,835 | 32.26 |  | 0.74 |  | 1,237 |  | 29 |  |
|  | 2 vehicles | 6,319 | 20.14 |  | 52.15 |  | 1,273 |  | 3,295 |  |
|  | 3+ vehicles | 3,766 | 50.72 |  | 42.83 |  | 1,910 |  | 1,613 |  |
| Franklin | No vehicle | 38,414 | 32.62 | 15.41 | 16.59 | 7.82 | 12,532 | 14,719 | 6,372 | 7,417 |
|  | 1 vehicle | 136,598 | 10.65 |  | 6.54 |  | 14,549 |  | 8,939 |  |
|  | 2 vehicles | 147,952 | 9.88 |  | 2.28 |  | 14,624 |  | 3,380 |  |
|  | 3+ vehicles | 55,759 | 29.86 |  | 19.61 |  | 16,650 |  | 10,936 |  |
| Licking | No vehicle | 3,090 | 42.23 | 15.09 | 133.67 | 37.49 | 1,305 | 2,036 | 4,131 | 6,025 |
|  | 1 vehicle | 13,901 | 7.96 |  | 3.79 |  | 1,107 |  | 527 |  |
|  | 2 vehicles | 19,644 | 12.56 |  | 43.67 |  | 2,468 |  | 8,578 |  |
|  | 3+ vehicles | 10,619 | 21.21 |  | 42.20 |  | 2,252 |  | 4,481 |  |
| Madison | No vehicle | 210 | 31.03 | 25.99 | 138.89 | 49.38 | 65 | 350 | 291 | 590 |
|  | 1 vehicle | 1,019 | 3.08 |  | 7.06 |  | 31 |  | 72 |  |
|  | 2 vehicles | 1,539 | 17.45 |  | 36.39 |  | 268 |  | 560 |  |
|  | 3+ vehicles | 950 | 63.29 |  | 96.04 |  | 601 |  | 912 |  |
| Pickaway | No vehicle | 249 | 63.89 | 12.85 | 565.86 | 121.34 | 159 | 161 | 1,409 | 1,261 |
|  | 1 vehicle | 1,173 | 3.51 |  | 132.42 |  | 41 |  | 1,553 |  |
|  | 2 vehicles | 1,785 | 1.42 |  | 22.21 |  | 25 |  | 396 |  |
|  | 3+ vehicles | 1,006 | 31.41 |  | 174.28 |  | 316 |  | 1,753 |  |
| Union | No vehicle | 186 | 140.59 | 82.69 | 554.21 | 115.67 | 261 | 914 | 1,030 | 1,119 |
|  | 1 vehicle | 954 | 83.92 |  | 128.32 |  | 800 |  | 1,224 |  |
|  | 2 vehicles | 1,557 | 64.71 |  | 14.22 |  | 1,008 |  | 222 |  |
|  | 3+ vehicles | 945 | 99.69 |  | 183.83 |  | 943 |  | 1,738 |  |
| Overall weighted mean error |  |  | 16.64 |  | 15.82 |  | 13,160 |  | 6,951 |  |

TABLE 1b Vehicle Ownership Level by County – Comparison with the Census Data (Year 2000)

| County | Vehicle ownership level (VOL) | Number of households in vehicle ownership level from Census | Absolute Percentage Error (APE) |  |  |  | Root Squared Error (RSE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip-based model |  | Tour-based model |  | Trip-based model |  | Tour-based model |  |
|  |  |  | By VOL | Wtd.Mean | By VOL | Wtd.Mean | By VOL | Wtd.Mean | By VOL | Wtd.Mean |
| Delaware | No vehicle | 1,153 | 130.44 | 27.08 | 197.49 | 49.88 | 1,504 | 4,047 | 2,277 | 7,574 |
|  | 1 vehicle | 8,576 | 10.40 |  | 66.48 |  | 892 |  | 5,702 |  |
|  | 2 vehicles | 20,294 | 25.50 |  | 48.33 |  | 5,174 |  | 9,808 |  |
|  | 3+ vehicles | 9,651 | 32.88 |  | 20.75 |  | 3,173 |  | 2,003 |  |
| Fairfield | No vehicle | 846 | 1.49 | 18.26 | 140.57 | 47.34 | 13 | 1,481 | 1,189 | 2,800 |
|  | 1 vehicle | 4,660 | 9.24 |  | 27.99 |  | 431 |  | 1,304 |  |
|  | 2 vehicles | 7,855 | 0.35 |  | 46.33 |  | 27 |  | 3,639 |  |
|  | 3+ vehicles | 4,810 | 59.20 |  | 51.35 |  | 2,847 |  | 2,470 |  |
| Franklin | No vehicle | 37,656 | 55.13 | 15.41 | 13.06 | 7.08 | 20,761 | 16,448 | 4,918 | 7,828 |
|  | 1 vehicle | 168,620 | 10.15 |  | 4.96 |  | 17,121 |  | 8,365 |  |
|  | 2 vehicles | 171,804 | 8.93 |  | 2.58 |  | 15,346 |  | 4,440 |  |
|  | 3+ vehicles | 60,698 | 23.73 |  | 21.97 |  | 14,401 |  | 13,333 |  |
| Licking | No vehicle | 3,408 | 25.06 | 9.76 | 161.98 | 31.96 | 854 | 1,763 | 5,520 | 6,158 |
|  | 1 vehicle | 15,580 | 2.82 |  | 1.59 |  | 439 |  | 248 |  |
|  | 2 vehicles | 23,152 | 9.92 |  | 38.97 |  | 2,296 |  | 9,022 |  |
|  | 3+ vehicles | 13,469 | 13.63 |  | 22.13 |  | 1,836 |  | 2,980 |  |
| Madison | No vehicle | 265 | 7.28 | 24.48 | 182.43 | 49.35 | 19 | 367 | 483 | 587 |
|  | 1 vehicle | 1,159 | 8.99 |  | 16.79 |  | 104 |  | 195 |  |
|  | 2 vehicles | 1,732 | 18.79 |  | 36.84 |  | 325 |  | 638 |  |
|  | 3+ vehicles | 1,083 | 54.37 |  | 71.66 |  | 589 |  | 776 |  |
| Pickaway | No vehicle | 232 | 17.85 | 17.30 | 1085.47 | 123.72 | 41 | 310 | 2,515 | 1,230 |
|  | 1 vehicle | 1,245 | 5.38 |  | 91.81 |  | 67 |  | 1,143 |  |
|  | 2 vehicles | 2,040 | 6.60 |  | 55.98 |  | 135 |  | 1,142 |  |
|  | 3+ vehicles | 1,235 | 46.92 |  | 87.38 |  | 579 |  | 1,079 |  |
| Union | No vehicle | 193 | 186.97 | 95.16 | 853.41 | 126.87 | 361 | 1,374 | 1,648 | 1,468 |
|  | 1 vehicle | 1,115 | 111.85 |  | 165.02 |  | 1,248 |  | 1,841 |  |
|  | 2 vehicles | 2,094 | 69.04 |  | 24.49 |  | 1,446 |  | 513 |  |
|  | 3+ vehicles | 1,332 | 108.93 |  | 150.60 |  | 1,450 |  | 2,005 |  |
| Overall weighted mean error |  |  | 16.52 |  | 16.11 |  | 14,536 |  | 7,453 |  |

TABLE 1c Vehicle Ownership Level by County – Comparison with the HIS Data (Year 2000)

| County | Vehicle ownership level (VOL) | Number of households in vehicle ownership level from HIS | Absolute Percentage Error (APE) |  |  |  | Root Squared Error (RSE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip-based model |  | Tour-based model |  | Trip-based model |  | Tour-based model |  |
|  |  |  | By VOL | Wtd. Mean | By VOL | Wtd.Mean | By VOL | Wtd.Mean | By VOL | Wtd. Mean |
| Delaware | No vehicle | 0 | - | 29.47 | - | 42.14 | 2,657 | 2,623 | 3,430 | 4,166 |
|  | 1 vehicle | 4,719 | 100.64 |  | 202.56 |  | 4,749 |  | 9,559 |  |
|  | 2 vehicles | 12,813 | 18.01 |  | 18.16 |  | 2,307 |  | 2,327 |  |
|  | 3+ vehicles | 11,364 | 12.85 |  | 2.55 |  | 1,460 |  | 290 |  |
| Fairfield | No vehicle | 0 | - | 93.48 | - | 76.67 | 833 | 3,376 | 2,034 | 2,742 |
|  | 1 vehicle | 1,832 | 130.84 |  | 225.53 |  | 2,397 |  | 4,132 |  |
|  | 2 vehicles | 4,487 | 74.46 |  | 6.03 |  | 3,341 |  | 271 |  |
|  | 3+ vehicles | 3,870 | 97.86 |  | 88.10 |  | 3,787 |  | 3,409 |  |
| Franklin | No vehicle | 40,236 | 45.19 | 14.01 | 18.64 | 9.99 | 18,181 | 12,195 | 7,498 | 14,274 |
|  | 1 vehicle | 158,956 | 4.69 |  | 11.34 |  | 7,457 |  | 18,029 |  |
|  | 2 vehicles | 162,742 | 3.86 |  | 8.30 |  | 6,284 |  | 13,502 |  |
|  | 3+ vehicles | 49,410 | 51.99 |  | 4.14 |  | 25,689 |  | 2,045 |  |
| Licking | No vehicle | 2,868 | 48.61 | 9.42 | 211.30 | 34.53 | 1,394 | 1,310 | 6,060 | 5,477 |
|  | 1 vehicle | 14,715 | 2.90 |  | 7.56 |  | 426 |  | 1,113 |  |
|  | 2 vehicles | 21,886 | 4.71 |  | 35.44 |  | 1,030 |  | 7,756 |  |
|  | 3+ vehicles | 13,193 | 16.01 |  | 24.68 |  | 2,112 |  | 3,256 |  |
| Madison | No vehicle | 114 | 149.12 | 192.46 | 555.88 | 180.17 | 170 | 1,097 | 634 | 865 |
|  | 1 vehicle | 217 | 482.03 |  | 523.68 |  | 1,046 |  | 1,136 |  |
|  | 2 vehicles | 747 | 175.37 |  | 46.43 |  | 1,310 |  | 347 |  |
|  | 3+ vehicles | 726 | 130.30 |  | 156.11 |  | 946 |  | 1,133 |  |
| Pickaway | No vehicle | 0 | - | 91.10 | - | 184.22 | 273 | 899 | 2,746 | 1,730 |
|  | 1 vehicle | 760 | 72.63 |  | 214.20 |  | 552 |  | 1,628 |  |
|  | 2 vehicles | 900 | 141.67 |  | 253.63 |  | 1,275 |  | 2,283 |  |
|  | 3+ vehicles | 1,114 | 62.84 |  | 107.69 |  | 700 |  | 1,200 |  |
| Union | No vehicle | 0 | - | 2.23 | - | 25.60 | 554 | 69 | 1,841 | 785 |
|  | 1 vehicle | 2,244 | 5.30 |  | 31.73 |  | 119 |  | 712 |  |
|  | 2 vehicles | 3,583 | 1.20 |  | 27.24 |  | 43 |  | 976 |  |
|  | 3+ vehicles | 2,813 | 1.10 |  | 18.62 |  | 31 |  | 524 |  |
| Overall weighted mean error |  |  | 16.81 |  | 17.40 |  | 10,921 |  | 12,905 |  |

TABLE 1d Vehicle Ownership Level by County – Comparison with the ACS Data (Year 2005)

| County | Vehicle ownership level (VOL) | Number of households in vehicle ownership level from ACS | Absolute Percentage Error (APE) |  |  |  | Root Squared Error (RSE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip-based model |  | Tour-based model |  | Trip-based model |  | Tour-based model |  |
|  |  |  | By VOL | Wtd. Mean | By VOL | Wtd. Mean | By VOL | Wtd. Mean | By VOL | Wtd. Mean |
| Delaware | No vehicle | 1,040 | 187.12 | 23.69 | 153.13 | 56.64 | 1,946 | 5,124 | 1,593 | 11,991 |
|  | 1 vehicle | 12,325 | 0.20 |  | 72.51 |  | 25 |  | 8,937 |  |
|  | 2 vehicles | 26,856 | 24.71 |  | 57.74 |  | 6,637 |  | 15,508 |  |
|  | 3+ vehicles | 13,196 | 30.65 |  | 31.97 |  | 4,044 |  | 4,219 |  |
| Fairfield | No vehicle | 910 | 9.01 | 17.86 | 184.12 | 56.18 | 82 | 1,675 | 1,676 | 3,345 |
|  | 1 vehicle | 4,624 | 4.16 |  | 58.65 |  | 193 |  | 2,712 |  |
|  | 2 vehicles | 8,813 | 0.58 |  | 46.45 |  | 51 |  | 4,093 |  |
|  | 3+ vehicles | 5,390 | 59.35 |  | 48.38 |  | 3,199 |  | 2,608 |  |
| Franklin | No vehicle | 31,839 | 96.97 | 14.03 | 7.35 | 11.19 | 30,874 | 12,929 | 2,339 | 17,722 |
|  | 1 vehicle | 166,746 | 2.07 |  | 8.76 |  | 3,448 |  | 14,603 |  |
|  | 2 vehicles | 181,284 | 6.14 |  | 12.84 |  | 11,139 |  | 23,272 |  |
|  | 3+ vehicles | 67,010 | 25.70 |  | 14.62 |  | 17,224 |  | 9,799 |  |
| Licking | No vehicle | 2,958 | 60.55 | 11.46 | 243.85 | 35.73 | 1,791 | 2,089 | 7,213 | 7,707 |
|  | 1 vehicle | 14,696 | 6.50 |  | 14.39 |  | 955 |  | 2,115 |  |
|  | 2 vehicles | 24,432 | 12.18 |  | 47.56 |  | 2,975 |  | 11,621 |  |
|  | 3+ vehicles | 17,174 | 6.22 |  | 1.31 |  | 1,068 |  | 225 |  |
| Overall weighted mean error |  |  | 14.79 |  | 19.43 |  | 11,486 |  | 16,186 |  |

TABLE 2a Work Flow Distribution by County – Comparison with the Census Data (Year 1990)

| Origin county | Destination |  |  |  |  |  |  |  |  |  | Overall Weighted Mean Absolute Percentage Error (OWMAPE) by origin county |  | Overall Weighted Mean Root Squared Error (OWMRSE) by origin county |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Within origin county |  |  |  |  | Outside origin county |  |  |  |  |  |  |  |  |
|  | Census flow (in 1000s) | Absolute Percentage Error (APE) |  | Root Squared Error (RSE) |  | Census flow (in 1000s) | Absolute Percentage Error (APE) |  | Root Squared Error (RSE) |  |  |  |  |  |
|  |  | Trip-based model | Tour-based model | Trip-based model | Tour-based model |  | Trip-based model | Tour-based model | Trip-based model | Tour-based model | Trip-based model | Tour-based model | Trip-based model | Tour-based model |
| Delaware | 14.00 | 80.97 | 87.33 | 11,336 | 12,226 | 18.90 | 55.97 | 51.23 | 10,579 | 9,682 | 66.61 | 66.59 | 10,907 | 10,838 |
| Fairfield | 9.24 | 85.44 | 91.32 | 7,895 | 8,438 | 9.84 | 97.97 | 94.82 | 9,640 | 9,330 | 91.90 | 93.12 | 8,838 | 8,909 |
| Franklin | 464.10 | 1.69 | 0.83 | 7,833 | 3,861 | 20.20 | 74.45 | 61.27 | 15,038 | 12,377 | 4.72 | 3.35 | 8,260 | 4,547 |
| Licking | 39.40 | 12.64 | 10.04 | 4,978 | 3,957 | 19.00 | 28.06 | 6.26 | 5,332 | 1,190 | 17.65 | 8.81 | 5,096 | 3,320 |
| Madison | 2.17 | 100.00 | 100.00 | 2,170 | 2,170 | 2.85 | 140.69 | 126.39 | 4,013 | 3,605 | 123.11 | 114.99 | 3,343 | 3,068 |
| Pickaway | 2.65 | 100.00 | 100.00 | 2,646 | 2,646 | 2.62 | 137.77 | 126.00 | 3,608 | 3,300 | 118.79 | 112.93 | 3,161 | 2,989 |
| Union | 3.14 | 100.00 | 100.00 | 3,135 | 3,135 | 1.75 | 394.63 | 346.86 | 6,902 | 6,067 | 205.51 | 188.40 | 4,834 | 4,415 |
| Total flow/overall weighted mean error | 534.70 | 7.48 | 6.81 | 7,722 | 4,399 | 75.16 | 73.33 | 60.60 | 10,510 | 8,844 | 15.59 | 13.44 | 8,117 | 5,158 |

TABLE 2b Work Flow Distribution by County – Comparison with the Census Data (Year 2000)

| Origin county | Destination |  |  |  |  |  |  |  |  |  | Overall Weighted Mean Absolute Percentage Error (OWMAPE) by origin county |  | Overall Weighted Mean Root Squared Error (OWMRSE) by origin county |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Within origin county |  |  |  |  | Outside origin county |  |  |  |  |  |  |  |  |
|  | Census flow (in 1000s) | Absolute Percentage Error (APE) |  | Root Squared Error (RSE) |  | Census flow (in 1000s) | Absolute Percentage Error (APE) |  | Root Squared Error (RSE) |  |  |  |  |  |
|  |  | Trip-based model | Tour-based model | Trip-based model | Tour-based model |  | Trip-based model | Tour-based model | Trip-based model | Tour-based model | Trip-based model | Tour-based model | Trip-based model | Tour-based model |
| Delaware | 21.10 | 1.87 | 16.93 | 395 | 3,572 | 36.30 | 6.23 | 1.05 | 2,260 | 382 | 4.63 | 6.89 | 1,813 | 2,187 |
| Fairfield | 10.68 | 56.51 | 71.16 | 6,035 | 7,600 | 13.36 | 86.41 | 91.65 | 11,544 | 12,244 | 73.13 | 82.55 | 9,500 | 10,439 |
| Franklin | 508.40 | 0.32 | 1.70 | 1,624 | 8,658 | 37.30 | 31.38 | 11.64 | 11,704 | 4,340 | 2.44 | 2.38 | 3,438 | 8,433 |
| Licking | 42.40 | 15.47 | 17.75 | 6,560 | 7,525 | 28.40 | 28.92 | 12.13 | 8,212 | 3,445 | 20.86 | 15.49 | 7,268 | 6,218 |
| Madison | 2.36 | 38.60 | 65.50 | 909 | 1,543 | 3.16 | 79.64 | 93.18 | 2,518 | 2,946 | 62.12 | 81.36 | 1,997 | 2,448 |
| Pickaway | 2.65 | 25.96 | 57.28 | 687 | 1,516 | 3.24 | 60.23 | 80.77 | 1,951 | 2,617 | 44.82 | 70.21 | 1,519 | 2,192 |
| Union | 3.86 | 60.56 | 37.66 | 2,338 | 1,454 | 2.87 | 100.19 | 122.07 | 2,876 | 3,505 | 77.46 | 73.66 | 2,581 | 2,540 |
| Total flow/overall weighted mean error | 591.45 | 3.14 | 5.39 | 2,461 | 8,368 | 124.63 | 32.95 | 23.65 | 8,520 | 5,014 | 8.33 | 8.57 | 4,200 | 7,887 |

TABLE 2c Work Trip Flow Distribution by County – Comparison with the HIS Data (Year 2000)

| Origin county | Destination county | HIS flow (in 1000s of trips) | Absolute Percentage Error (APE) |  |  |  | Root Squared Error (RSE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip-based model |  | Tour-based model |  | Trip-based model |  | Tour-based model |  |
|  |  |  | Destination county | Wtd. Mean | Destination county | Wtd. Mean | Destination county | Wtd. Mean | Destination county | Wtd. Mean |
| Delaware | Delaware | 13.08 | 47.59 | 24.08 | 25.34 | 11.80 | 6,223 | 4,979 | 3,313 | 2,136 |
|  | Fairfield | 0.00 | - |  | - |  | 54 |  | 58 |  |
|  | Franklin | 33.23 | 13.57 |  | 4.48 |  | 4,508 |  | 1,489 |  |
|  | Licking | 0.25 | 183.37 |  | 163.00 |  | 467 |  | 415 |  |
|  | Madison | 0.00 | - |  | - |  | 208 |  | 272 |  |
|  | Pickaway | 0.00 | - |  | - |  | 27 |  | 42 |  |
|  | Union | 1.12 | 25.10 |  | 36.52 |  | 280 |  | 408 |  |
| Fairfield | Delaware | 0.39 | 38.12 | 79.17 | 38.93 | 96.77 | 149 | 6,880 | 152 | 8,001 |
|  | Fairfield | 4.85 | 14.09 |  | 40.67 |  | 684 |  | 1,974 |  |
|  | Franklin | 10.03 | 86.53 |  | 99.86 |  | 8,681 |  | 10,018 |  |
|  | Licking | 0.19 | 1409.89 |  | 1512.40 |  | 2,633 |  | 2,824 |  |
|  | Madison | 0.59 | 95.67 |  | 96.08 |  | 562 |  | 564 |  |
|  | Pickaway | 0.00 | - |  | - |  | 543 |  | 612 |  |
|  | Union | 0.00 | - |  | - |  | 19 |  | 8 |  |
| Franklin | Delaware | 20.54 | 38.97 | 7.07 | 16.33 | 4.70 | 8,003 | 24,379 | 3,353 | 15,249 |
|  | Fairfield | 1.58 | 35.10 |  | 95.45 |  | 554 |  | 1,508 |  |
|  | Franklin | 483.00 | 5.19 |  | 3.25 |  | 25,053 |  | 15,689 |  |
|  | Licking | 2.94 | 31.95 |  | 30.65 |  | 939 |  | 901 |  |
|  | Madison | 1.21 | 26.33 |  | 90.13 |  | 319 |  | 1,093 |  |
|  | Pickaway | 0.75 | 80.56 |  | 194.44 |  | 605 |  | 1,461 |  |
|  | Union | 2.29 | 32.26 |  | 4.13 |  | 738 |  | 94 |  |

TABLE 2c (continued) Work Trip Flow Distribution by County – Comparison with the HIS Data (Year 2000)

| Origin county | Destination county | HIS flow (in 1000s of trips) | Absolute Percentage Error (APE) |  |  |  | Root Squared Error (RSE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip-based model |  | Tour-based model |  | Trip-based model |  | Tour-based model |  |
|  |  |  | Destination county | Wtd. Mean | Destination county | Wtd. Mean | Destination county | Wtd. Mean | Destination county | Wtd. Mean |
| Licking | Delaware | 0.91 | 2.17 | 10.43 | 68.57 | 14.18 | 20 | 3,880 | 624 | 3,162 |
|  | Fairfield | 0.55 | 28.02 |  | 149.76 |  | 155 |  | 827 |  |
|  | Franklin | 14.21 | 15.39 |  | 42.47 |  | 2,187 |  | 6,036 |  |
|  | Licking | 48.27 | 8.92 |  | 3.28 |  | 4,305 |  | 1,581 |  |
|  | Madison | 0.00 | - |  | - |  | 26 |  | 20 |  |
|  | Pickaway | 0.00 | - |  | - |  | 61 |  | 108 |  |
|  | Union | 0.00 | - |  | - |  | 43 |  | 45 |  |
| Madison | Delaware | 0.25 | 40.16 | 118.70 | 5.62 | 143.62 | 101 | 2,217 | 14 | 2,523 |
|  | Fairfield | 0.00 | - |  | - |  | 8 |  | 5 |  |
|  | Franklin | 1.45 | 218.27 |  | 247.02 |  | 3,175 |  | 3,593 |  |
|  | Licking | 0.00 | - |  | - |  | 13 |  | 14 |  |
|  | Madison | 1.20 | 8.36 |  | 36.59 |  | 100 |  | 439 |  |
|  | Pickaway | 0.00 | - |  | - |  | 49 |  | 84 |  |
|  | Union | 0.08 | 206.14 |  | 296.87 |  | 169 |  | 243 |  |
| Pickaway | Delaware | 0.00 | - | 98.65 | - | 98.63 | 43 | 1,710 | 38 | 2,322 |
|  | Fairfield | 0.10 | 40.63 |  | 7.53 |  | 42 |  | 8 |  |
|  | Franklin | 2.62 | 70.83 |  | 99.83 |  | 1,856 |  | 2,616 |  |
|  | Licking | 0.00 | - |  | - |  | 41 |  | 30 |  |
|  | Madison | 0.16 | 83.71 |  | 68.22 |  | 132 |  | 107 |  |
|  | Pickaway | 0.48 | 269.41 |  | 121.97 |  | 1,283 |  | 581 |  |
|  | Union | 0.00 | - |  | - |  | 13 |  | 10 |  |
| Union | Delaware | 0.55 | 13.85 | 29.55 | 141.14 | 31.76 | 76 | 1,076 | 773 | 843 |
|  | Fairfield | 0.00 | - |  | - |  | 3 |  | 1 |  |
|  | Franklin | 4.01 | 4.20 |  | 7.09 |  | 168 |  | 284 |  |
|  | Licking | 0.00 | - |  | - |  | 13 |  | 17 |  |
|  | Madison | 1.85 | 81.61 |  | 82.55 |  | 1,511 |  | 1,528 |  |
|  | Pickaway | 0.00 | - |  | - |  | 7 |  | 10 |  |
|  | Union | 4.19 | 32.85 |  | 18.62 |  | 1,376 |  | 780 |  |
| Total flow/overall weighted mean error |  | 656.92 | 11.73 |  | 9.94 |  | 21,633 |  | 13,575 |  |

TABLE 2d Work Flow Distribution by County – Comparison with the ACS Data (Year 2005)

| Origin county | Destination |  |  |  |  |  |  |  |  |  | Overall Weighted Mean Absolute Percentage Error (OWMAPE) by origin county |  | Overall Weighted Mean Root Squared Error (OWMRSE) by origin county |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Within origin county |  |  |  |  | Outside origin county |  |  |  |  |  |  |  |  |
|  | ACS flow (in 1000s) | Absolute Percentage Error (APE) |  | Root Squared Error (RSE) |  | ACS flow (in 1000s) | Absolute Percentage Error (APE) |  | Root Squared Error (RSE) |  |  |  |  |  |
|  |  | Trip-based model | Tour-based model | Trip-based model | Tour-based model |  | Trip-based model | Tour-based model | Trip-based model | Tour-based model | Trip-based model | Tour-based model | Trip-based model | Tour-based model |
| Delaware | 28.29 | 4.67 | 21.52 | 1,321 | 6,088 | 45.73 | 9.78 | 3.76 | 4,472 | 1,719 | 7.83 | 10.55 | 3,609 | 3,999 |
| Fairfield | 12.36 | 55.08 | 69.58 | 6,808 | 8,602 | 14.51 | 69.94 | 78.91 | 10,151 | 11,453 | 63.10 | 74.62 | 8,773 | 10,241 |
| Franklin | 471.31 | 6.49 | 4.45 | 30,579 | 20,959 | 44.68 | 42.64 | 28.28 | 19,051 | 12,635 | 9.62 | 6.51 | 29,758 | 20,373 |
| Licking | 40.82 | 4.93 | 4.84 | 2,013 | 1,974 | 32.90 | 21.47 | 4.24 | 7,065 | 1,395 | 12.31 | 4.57 | 4,951 | 1,740 |
| Total flow/overall weighted mean error | 552.78 | 7.37 | 6.81 | 28,261 | 19,452 | 137.82 | 29.56 | 19.74 | 12,127 | 8,186 | 11.80 | 9.39 | 25,858 | 17,783 |

TABLE 3 Work Flow Distribution by Trip Start Time – Comparison with the Census/ACS Data

| Origin county | Peak period |  |  |  |  |  |  |  | Off-peak period |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Census/ACS flow (in 1000s) |  | Absolute Percentage Error (APE) |  | Root Squared Error (RSE) |  |  |  | Census/ACs flow (in 1000s) |  | Absolute Percentage Error (APE) |  |  | Root Squared Error (RSE) |  |
|  |  |  | Trip-based model | Tour-based model | Trip-based model | Tour-based model |  |  |  |  | Trip-based model |  | Tour-based model | Trip-based model | Tour-based model |
| Comparison with the Census Data - Year 1990 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Delaware | 22.37 |  | 10.93 | 18.71 | 2,446 | 4,185 |  |  | 10.53 |  | 16.04 |  | 15.59 | 1,689 | 1,641 |
| Fairfield | 12.00 |  | 7.59 | 0.76 | 911 | 91 |  |  | 7.08 |  | 11.79 |  | 13.90 | 835 | 984 |
| Franklin | 336.38 |  | 12.07 | 9.71 | 40,587 | 32,646 |  |  | 147.92 |  | 22.57 |  | 16.31 | 33,381 | 24,131 |
| Licking | 36.00 |  | 0.03 | 9.89 | 12 | 3,562 |  |  | 22.40 |  | 1.52 |  | 7.08 | 341 | 1,586 |
| Madison | 3.15 |  | 35.07 | 19.29 | 1,105 | 608 |  |  | 1.87 |  | 39.41 |  | 44.20 | 737 | 827 |
| Pickaway | 3.24 |  | 19.13 | 6.37 | 620 | 206 |  |  | 2.02 |  | 16.90 |  | 22.10 | 342 | 447 |
| Union | 2.89 |  | 85.66 | 59.39 | 2,475 | 1,716 |  |  | 2.00 |  | 64.77 |  | 60.94 | 1,292 | 1,216 |
| Overall weighted mean error |  |  | 11.57 | 10.34 | 36,500 | 29,390 |  |  | - |  | 19.93 |  | 15.91 | 29,166 | 21,093 |
| Comparison with the Census Data - Year 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Delaware |  | 41.13 | 16.30 | 18.16 | 6,703 |  | 7,468 |  |  | 16.27 | 29.75 | 26.31 |  | 4,838 | 4,279 |
| Fairfield |  | 14.88 | 23.15 | 18.78 | 3,444 |  | 2,794 |  |  | 9.16 | 22.53 | 20.20 |  | 2,065 | 1,851 |
| Franklin |  | 366.76 | 9.45 | 7.31 | 34,671 |  | 26,808 |  |  | 178.94 | 13.74 | 7.72 |  | 24,591 | 13,811 |
| Licking |  | 42.70 | 0.39 | 8.93 | 168 |  | 3,813 |  |  | 28.10 | 6.48 | 0.95 |  | 1,821 | 267 |
| Madison |  | 3.32 | 33.09 | 26.39 | 1,099 |  | 876 |  |  | 2.20 | 23.21 | 23.97 |  | 510 | 527 |
| Pickaway |  | 3.51 | 26.37 | 15.93 | 925 |  | 559 |  |  | 2.38 | 14.27 | 22.82 |  | 339 | 543 |
| Union |  | 3.98 | 85.99 | 81.15 | 3,424 |  | 3,232 |  |  | 2.75 | 65.10 | 62.80 |  | 1,790 | 1,727 |
| Overall weighted mean error |  |  | 10.59 | 9.56 | 30,496 |  | 23,662 |  |  | - | 14.99 | 9.59 |  | 21,294 | 11,990 |
| Comparison with the ACS Data - Year 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Delaware | 52.40 |  | 19.28 | 21.03 | 10,104 |  |  | 11,022 |  | 21.61 | 19.95 |  | 14.88 | 4,311 | 3,216 |
| Fairfield | 16.03 |  | 16.87 | 15.90 | 2,705 |  |  | 2,549 |  | 10.84 | 5.88 |  | 2.79 | 637 | 302 |
| Franklin | 347.34 |  | 5.84 | 3.75 | 20,275 |  |  | 13,036 |  | 168.66 | 18.86 |  | 12.67 | 31,803 | 21,360 |
| Licking | 43.79 |  | 8.47 | 1.18 | 3,707 |  |  | 515 |  | 29.94 | 17.94 |  | 9.53 | 5,370 | 2,854 |
| Overall weighted mean error |  |  | 8.01 | 5.90 | 17,997 |  |  | 11,939 |  | - | 18.23 |  | 12.00 | 27,272 | 18,305 |

TABLE 4 Travel Time for Work Trips – Comparison with the Census Data (Year 1990)

| Origin county | Census/ACS average travel time from origin county (in mins) | Absolute Percentage Error (APE) |  |  | Root Squared Error (RSE) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Trip-based model | Tour-based model | Trip-based model |  | Tour-based model |
| Comparison with the Census Data - Year 1990 |  |  |  |  |  |  |
| Delaware | 22.74 | 28.88 | 30.70 | 6.57 |  | 6.98 |
| Fairfield | 24.75 | 10.94 | 16.19 | 2.71 |  | 4.01 |
| Franklin | 20.04 | 37.61 | 16.36 | 7.54 |  | 3.28 |
| Licking | 22.1 | 28.87 | 10.48 | 6.38 |  | 2.32 |
| Madison | 22.75 | 8.04 | 10.96 | 1.83 |  | 2.49 |
| Pickaway | 23.72 | 15.83 | 17.41 | 3.76 |  | 4.13 |
| Union | 20.97 | 50.82 | 46.83 | 10.66 |  | 9.82 |
| Overall weighted mean error |  | 35.14 | 16.77 | 7.25 |  | 3.63 |
| Comparison with the Census Data - Year 2000 |  |  |  |  |  |  |
| Delaware | 25.45 | 20.46 | 19.38 | 5.21 |  | 4.93 |
| Fairfield | 26.95 | 1.73 | 2.19 | 0.47 |  | 0.59 |
| Franklin | 21.41 | 26.74 | 21.53 | 5.73 |  | 4.61 |
| Licking | 24.12 | 20.32 | 22.70 | 4.90 |  | 5.48 |
| Madison | 25.01 | 7.11 | 7.15 | 1.78 |  | 1.79 |
| Pickaway | 26.19 | 9.13 | 4.72 | 2.39 |  | 1.24 |
| Union | 22.29 | 15.80 | 17.66 | 3.52 |  | 3.94 |
| Overall weighted mean error |  | 24.36 | 20.54 | 5.45 |  | 4.62 |
| Comparison with the ACS Data - Year 2005 |  |  |  |  |  |  |
| Delaware | 22.87 | 11.16 | 10.88 | 2.55 |  | 2.49 |
| Fairfield | 25.04 | 3.08 | 5.68 | 0.77 |  | 1.42 |
| Franklin | 18.76 | 15.93 | 11.69 | 2.99 |  | 2.19 |
| Licking | 24.28 | 9.02 | 11.67 | 2.19 |  | 2.83 |
| Overall weighted mean error |  | 14.18 | 11.37 | 2.81 |  | 2.28 |

TABLE 5 Project Level Link Volume Comparison by Roadway Functional Class

| Study project | Year | Roadway functional class | Survey data |  | Absolute Percentage Error (APE) |  |  |  | Percentage Root Mean Squared Error (%RMSE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Trip-based model |  | Tour-based model |  | Trip-based model |  | Tour-based model |  |
|  |  |  | Number of links | Total link flow (vehs/day) | By roadway functional class | Wtd. Mean | By roadway functional class | Wtd. Mean | By roadway functional class | Wtd. Mean | By roadway functional class | Wtd. Mean |
| Polaris | 1990 | Freeway (interstate) | 2 | 40,400 | 20.73 | 41.82 | 6.01 | 32.09 | 30.73 | 57.09 | 8.87 | 45.15 |
|  |  | Major roads (arterials) | 6 | 9,410 | 81.36 |  | 93.69 |  | 93.87 |  | 112.97 |  |
|  |  | Minor roads (collectors) | 10 | 23,724 | 62.07 |  | 52.06 |  | 87.37 |  | 80.03 |  |
|  | 2000 | Freeway (interstate) | 4 | 168,300 | 10.45 | 22.13 | 8.45 | 23.31 | 12.92 | 29.22 | 11.88 | 28.59 |
|  |  | Major roads (arterials) | 16 | 233,980 | 29.19 |  | 34.63 |  | 38.87 |  | 41.46 |  |
|  |  | Minor roads (collectors) | 8 | 42,438 | 29.47 |  | 19.78 |  | 40.64 |  | 23.97 |  |
|  | 2005 | Freeway (interstate) | 8 | 369,782 | 14.58 | 23.33 | 12.63 | 22.16 | 22.69 | 31.50 | 14.79 | 28.92 |
|  |  | Major roads (arterials) | 26 | 343,350 | 28.27 |  | 28.28 |  | 35.92 |  | 39.26 |  |
|  |  | Minor roads (collectors) | 10 | 62,205 | 48.06 |  | 45.04 |  | 59.43 |  | 55.85 |  |
| Hilliard-Rome | 1990 | Freeway (interstate) | 8 | 223,220 | 13.79 | 22.14 | 7.22 | 21.08 | 15.76 | 27.15 | 8.54 | 26.66 |
|  |  | Major roads (arterials) | 42 | 388,064 | 22.27 |  | 24.90 |  | 26.41 |  | 30.70 |  |
|  |  | Minor roads (collectors) | 48 | 102,948 | 37.82 |  | 34.77 |  | 52.51 |  | 48.46 |  |
|  |  | Local roads | 4 | 2,808 | 92.04 |  | 94.44 |  | 106.36 |  | 109.09 |  |
|  | 2000 | Freeway (interstate) | 10 | 526,542 | 6.60 | 20.29 | 12.70 | 19.64 | 7.67 | 25.05 | 16.10 | 25.04 |
|  |  | Major roads (arterials) | 52 | 570,258 | 26.47 |  | 19.89 |  | 33.45 |  | 26.21 |  |
|  |  | Minor roads (collectors) | 58 | 236,552 | 31.27 |  | 30.13 |  | 38.17 |  | 37.12 |  |
|  |  | Local roads | 10 | 15,406 | 90.67 |  | 86.32 |  | 106.88 |  | 102.05 |  |
|  | 2005 | Freeway (interstate) | 10 | 556,698 | 10.98 | 21.98 | 8.45 | 19.14 | 13.37 | 28.39 | 13.22 | 25.22 |
|  |  | Major roads (arterials) | 65 | 803,945 | 24.07 |  | 19.57 |  | 32.20 |  | 25.26 |  |
|  |  | Minor roads (collectors) | 76 | 398,680 | 29.11 |  | 29.34 |  | 37.44 |  | 37.63 |  |
|  |  | Local roads | 14 | 23,206 | 90.90 |  | 85.58 |  | 101.51 |  | 98.23 |  |

TABLE 5 (continued) Project Level Link Volume Comparison by Roadway Functional Class

| Study project | Year | Roadway functional class | Survey data |  | Absolute Percentage Error (APE) |  |  |  | Percentage Root Mean Squared Error (%RMSE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Trip-based model |  | Tour-based model |  | Trip-based model |  | Tour-based model |  |
|  |  |  | Number of links | Total link flow (vehs/day) | By roadway functional class | Wtd. Mean | By roadway functional class | Wtd. Mean | By roadway functional class | Wtd. Mean | By roadway functional class | Wtd. Mean |
| Spring-Sandusky | 1990 | Freeway (interstate) | 39 | 1,704,739 | 11.95 | 32.62 | 24.87 | 35.73 | 15.20 | 41.71 | 29.71 | 45.40 |
|  |  | Expressway | 10 | 481,194 | 5.10 |  | 17.90 |  | 7.17 |  | 20.83 |  |
|  |  | On ramp | 1 | 26,759 | 0.58 |  | 0.80 |  | 0.00 |  | 0.00 |  |
|  |  | Major roads (arterials) | 412 | 3,992,054 | 39.13 |  | 37.69 |  | 49.70 |  | 48.48 |  |
|  |  | Minor roads (collectors) | 103 | 435,751 | 60.47 |  | 56.67 |  | 76.33 |  | 73.59 |  |
|  |  | Local roads | 68 | 197,460 | 89.57 |  | 91.80 |  | 122.53 |  | 122.45 |  |
|  | 2005 | Freeway (interstate) | 42 | 2,364,702 | 11.00 | 27.36 | 17.34 | 29.13 | 14.69 | 36.21 | 21.85 | 37.93 |
|  |  | Expressway | 10 | 448,944 | 25.45 |  | 31.50 |  | 34.19 |  | 41.45 |  |
|  |  | On ramp | 2 | 50,392 | 19.35 |  | 12.93 |  | 27.59 |  | 18.31 |  |
|  |  | Off ramp | 2 | 12,129 | 50.61 |  | 47.55 |  | 86.99 |  | 89.97 |  |
|  |  | Major roads (arterials) | 491 | 4,657,741 | 32.28 |  | 31.65 |  | 42.78 |  | 41.59 |  |
|  |  | Minor roads (collectors) | 135 | 530,099 | 47.45 |  | 47.06 |  | 60.34 |  | 58.36 |  |
|  |  | Local roads | 85 | 174,870 | 62.24 |  | 64.23 |  | 83.38 |  | 88.63 |  |
| Control Area | 1990 | Freeway (interstate) | 6 | 128,604 | 7.86 | 20.33 | 4.65 | 26.91 | 8.90 | 25.04 | 6.21 | 30.43 |
|  |  | Major roads (arterials) | 43 | 179,585 | 24.14 |  | 37.31 |  | 30.91 |  | 41.47 |  |
|  |  | Minor roads (collectors) | 14 | 16,496 | 66.18 |  | 78.09 |  | 71.45 |  | 83.59 |  |
|  |  | Local roads | 6 | 4,330 | 58.24 |  | 61.84 |  | 84.05 |  | 89.38 |  |
|  | 2000 | Freeway (interstate) | 6 | 175,990 | 7.23 | 16.27 | 7.18 | 17.97 | 8.48 | 19.97 | 8.33 | 22.37 |
|  |  | Major roads (arterials) | 30 | 161,454 | 20.55 |  | 24.67 |  | 26.27 |  | 31.46 |  |
|  |  | Minor roads (collectors) | 12 | 16,742 | 60.89 |  | 57.13 |  | 68.90 |  | 68.33 |  |
|  |  | Local roads | 13 | 2,854 | 69.58 |  | 74.90 |  | 84.50 |  | 105.30 |  |
|  | 2005 | Freeway (interstate) | 8 | 253,258 | 8.30 | 25.22 | 10.05 | 25.90 | 11.22 | 37.07 | 12.04 | 37.82 |
|  |  | Major roads (arterials) | 58 | 359,703 | 28.92 |  | 28.89 |  | 37.42 |  | 38.47 |  |
|  |  | Minor roads (collectors) | 24 | 67,545 | 60.19 |  | 58.24 |  | 116.94 |  | 114.99 |  |
|  |  | Local roads | 24 | 21,820 | 52.30 |  | 60.62 |  | 83.99 |  | 87.67 |  |
| Overall weighted mean error | 1990 | - | - | - | 31.26 |  | 34.01 |  | 39.85 |  | 43.09 |  |
|  | 2000 | - | - | - | 20.00 |  | 20.12 |  | 25.07 |  | 25.33 |  |
|  | 2005 | - | - | - | 26.13 |  | 26.91 |  | 34.73 |  | 35.34 |  |

Planning agencies within the United States that have developed a tour-based or an activity-based travel model include Portland METRO, New York NYMTC, Columbus MORPC, Sacramento SACOG, the Los Angeles SCAG, Denver DRCOG, and the San Francisco SFCTA. Planning agencies that are in the process of either moving toward or considering the move toward the activity-based modeling approach include ARC of Atlanta GA, NCTCOG of Dallas-Fort Worth TX, HGAC of Houston, TX, CMAP of Chicago, ILPSRC of Seattle WA, MAG of Phoenix AZ, El Paso MPO, and SBCAG of Santa Barbara CA. Also, the reader is referred to Bowman and Bradley (1) and Pinjari and Bhat (2) for a summary of the design features of several of the activity based models developed (or under development) for practice. In addition, there have been activity based models developed in the research community, which include TRANSIMS, ILUTE (3), CEMDAP (4-6), FAMOS (7,8), SimAGENT (9), and ALBATROSS (10).

In the current study, travel behavior is compared before projects, and again after projects, although changes in behavior from before to after are not compared due to data limitations. This issue is discussed in more detail in the final report submitted to ODOT (see Ferdous et al. (11)).

Future efforts should examine convergence criteria-related considerations carefully, since it is likely that several iterations will be needed to bring supply and demand to anything close to an equilibrium solution.

The year 2000 was included in the “before-after” project analysis because of the availability of the 2000 Census data, as well as the 1999 Household Interview Survey (HIS), that contributed toward our region-level analysis comparison of the trip-based and tour-based model system outputs. Further, for the local-level analysis, the year 2000 represented the completion of the Polaris Parkway widening (even though the I-71 split interchanges were not completed by then) and the immediate “after” situation for the Hilliard-Rome project.

The Census data are available for all seven counties under consideration here. However, the ACS data are available only for Delaware, Fairfield, Franklin, and Licking counties.

In the interest of brevity, results of the evaluation of district-level O-D work flow distributions within Franklin County (which is the dominant county in the study region) and average trip distance distribution (by trip type and by county of residence) are not presented in the current paper. The analysis of the district-level O-D work flow distributions within Franklin County indicated that the tour-based model significantly outperformed the trip-based model (when compared against estimates from the 2000 Census Transportation Planning Products or CTPP 2000). For the trip distance distribution, we are unable to compare the outputs from the model systems to the Census, the ACS, or the HIS because these data sets do not provide information on observed trip distances.

This immediately brings attention to the aggregate-level modeling approach of the MORPC trip-based model relative to the disaggregate-level modeling approach of the MORPC tour-based model. Note that the vehicle ownership model in the trip-based modeling framework is implemented for each TAZ using the Iterative Proportional Fitting (IPF) technique to predict household vehicle ownership level within each TAZ by household size and income group, while vehicle ownership is estimated at the household level (using a discrete choice model) and also applied at the household level in the activity-based modeling framework. It is important to emphasize that the comparison being undertaken in this project is between the aggregate-level trip-based and disaggregate-level tour-based modeling frameworks as represented in the MORPC efforts.

See Ferdous et al. (11) for a description of the steps undertaken to process the work flow outputs from the trip-based and the tour-based models.

For compactness, we have suppressed the results comparison with the HIS data in this section as well as in the next section (i.e., Section 4.1.4) of the current paper. These results are available in the ODOT final report (see Ferdous et al. (11)). Also, a time-of-day model component was developed for the trip-based model, with the same time periods and structure as the tour based model. The time periods were am peak (6:30 am-9:29 am), mid-day (9:30 am-3:29 pm), pm peak (3:30 pm-6:29 pm), and evening (6:30 pm-6:29 am).

In discussions with MORPC and ODOT staff, it was decided that, as the Spring-Sandusky project was incomplete in the year 2000, no comparison will be undertaken for this project for this year. However, as the Polaris project was undertaken in phases and the year 2000 marked the completion of the parkway widening phase of the project, we undertook a comparative analysis for the Polaris project for the year 2000 even though the entire project was not complete until 2007 (see also footnote 7).