### Distances on a curved earth

### GIS in Water Resources

### Prepared by David G. Tarboton, 
Utah State University, Updated September 2008

A common task working with geographic data is the calculation of the distance between two points on the surface of the earth.  For points close together the earth surface may be approximated as a plane using a convenient projection, but as points get further apart the approximation involved with a projection becomes less accurate and it is better to calculate the distance along an arc around the surface of the earth.  In performing this calculation the earth may be approximated as a spheroid, or if more precise results are required as an ellipsoid.  We will illustrate some of these methods to calculate the distance between points along the Hurricane Katrina track.  The following table gives locations of the center of Hurricane Katrina as it moved from the Atlantic, across Florida, the Gulf of Mexico then over New Orleans and into the US in August 2005.

| Date | Latitude | Longitude | Lat rad | Long Rad |
| --- | --- | --- | --- | --- |
|  |  |  |  |  |
| Wed Aug 24 10AM | 24.7000 | -76.7000 | 0.43110 | -1.33867 |
| Wed Aug 24 10PM | 26.0000 | -78.0000 | 0.45379 | -1.36136 |
| Thur Aug 25 10AM | 26.2000 | -79.3000 | 0.45728 | -1.38405 |
| Thur Aug 25 10PM | 25.5000 | -80.7000 | 0.44506 | -1.40848 |
| Fri Aug 26 10AM | 25.1000 | -82.2000 | 0.43808 | -1.43466 |
| Fri Aug 26 10PM | 24.6000 | -83.6000 | 0.42935 | -1.45910 |
| Sat Aug 27 10AM | 24.5000 | -85.0000 | 0.42761 | -1.48353 |
| Sat Aug 27 10PM | 25.0000 | -86.2000 | 0.43633 | -1.50447 |
| Sun Aug 28 10AM | 26.0000 | -88.1000 | 0.45379 | -1.53764 |
| Sun Aug 28 10PM | 27.6000 | -89.4000 | 0.48171 | -1.56032 |
| Mon Aug 29 10AM | 30.2000 | -89.6000 | 0.52709 | -1.56382 |
| Mon Aug 29 10PM | 33.5000 | -88.5000 | 0.58469 | -1.54462 |
| Tues Aug 30 10AM | 36.3000 | -87.5000 | 0.63355 | -1.52716 |

1.  Distance using a projection on to a flat plane.  A feature class KatrinaPath was created using the above coordinates and displayed together with US States in Arcmap:

Note that in the coordinate display at the bottom coordinates are expressed in latitude and longitude.  This lets you know that this map display is in terms of geographic coordinates.  This can be verified by right clicking on  the table of contents "Layers" label referred to as a data frame and selecting properties.

In the Properties dialog that appears select the coordinate system tab to see that the coordinate system is GCS_North_American_1983.

This means a Geographic Coordinate system using latitude and longitude coordinates relative to the North American Datum of 1983 (NAD83).  Click on Predefined/Projected/Continental/North America and select "USA Contiguous Equidistant Conic".  From the name "Equidistant" this is a projection that preserves distances well.  Click OK.

Your Map display should now adjust itself so that meters are shown in the coordinate display at the bottom.  The map is now being displayed using projected coordinates.

In projected coordinates it is meaningful to select the Measure tool and measure distances between points as shown above.  The distance between two points on the Hurricane track, such as the 8/28 10 pm and 8/29 10 am points shown can be read as 288117 m, or 288 km.  This distance is somewhat approximate due to inaccuracy in positioning the Measure tool precisely over the points.

To more precisely calculate this distance we need the coordinates of each of these points in projected coordinates.  To achieve this we first need to create a new feature class that is in projected coordinates.  Right click on Katerina path and select Data/Export data as shown below

Click on use the same coordinate system as the data frame and browse to output a feature class in the geodatabase named 'katrina.mdb' giving the new feature class the name KatrinaProj.  Click OK and click Yes to add the features to the map.

The new feature class now displayed on the map has its coordinates saved internally in projected coordinates rather than geographic coordinates.  To retrieve these coordinates so that distance calculations can be performed the "Add XY Coordinates" tool will be used.  Select Data Management|Features|Add XY Coordinates from ArcToolbox

Select the input fueature class as KatrinaProj and click OK

Open the attribute table for KatrinaProj and see that two fields named Point_X and Point_Y have been added.

Now, geometry can be used to more precisely calculate the distances between the points.  Between the points of 8/28 10 pm and 8/29 10 am the distance is:

659534−6193772+1240515−9544792=288841 m = 288.8 km.  This is a bit more precise than the measure distance obtained above.

#### 2.  Distance on a curved earth represented as a spheroid.

Refer to http://en.wikipedia.org/wiki/Spherical_distance  or (Longley et al., 2001) for a discussion of this.  Wikipedia gives the following formula for calculating the "Great Circle" distance between two points with latitude i and longitude i.

This formula is equivalent to the haversine formula linked at the above page, but expressed not quite as concisely using more conventional trigonometric functions.

Longley et al. give the following formula

d(δ1,ϕ1,δ2,ϕ2)=rcos−1(sinδ1sinδ2+cosδ1cosδ2cos(ϕ1−ϕ2))

Implementing both of these formula's in Excel for the two points above I get a distance of 289.58 km.  The radius of the earth was taken as 6367 km in the below and angles were converted to radians.  (This is an embeddded Excel object so you can see the formula's if you download the Word document).

This verifies that these formula's are equivalent

#### 3.  Distance on a curved earth represented as an ellipsoid.

The earth is better approximated as an ellipsoid than a spheroid.  MATLAB script exchange includes an implementation of a function for calculating distances on the WGS84 reference ellipsoid.  This is given at http://www.mathworks.com/matlabcentral/fileexchange/loadFile.do?objectId=5379&objectType=FILE.  Evaluating this function in MATLAB I get

vdist(27.6, -89.4, 30.2, -89.6)

ans =  2.8883e+005

This reports the distance between the two points as 288.83 km.  I believe this ellipsoid distance to be the most precise.  This is quite close to the projected coordinate distance, but not as close to the great circle distance.  Therefore I conclude that distances calculated in projected coordinates are more precise at this scale than great circle distances.

#### References

Longley, P. A., M. F. Goodchild, D. J. Maguire and D. W. Rind, (2001), Geographic Information Systems and Science, Wiley, 454 p.